Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	FUNDAMENTALS OF THERMAL FLUID SCIENCES		Time : 3 hours

Subject Code:	CL211			       						                	 Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define surface tension.

2.	What are laminar and turbulent flows?

3.	State zeroth law of thermodynamics.

4.	Define dryness fraction of steam.

5.	What is meant by natural convection?

6.	Define radiation.

7.	What are the modes of condensation?

8.	Define boiling.

9.	What is meant by indirect heat exchanger?

10.	What is fouling factor?

PART – B  (5 x 3 = 15 MARKS)

11.	Name the different types of fluids.

12.	State the second laws of thermodynamics.

13.	Sketch the boundary layer development over a flat plate.

14.	Explain the boiling heat transfer phenomena.

15.	How the shell and tube heat exchanger works?

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the Bernoulli’s equation of motion with their assumptions.

(OR)

17.	An oil of viscosity 0.1 Ns/m2 and relative density 0.9 is flowing through a circular pipe of diameter 50 mm and of length 300 m. The rate of flow of fluid through the pipe is 3.5 litres/s. Find the pressure drop in a length of 300 m and also the shear stress at the pipe wall.

18.	A wall of a cold room is composed of three layers. The outer layer is brick of 20 cm thick, the middle layer is cork of 10 cm thick, the inside layer is cement of 5 cm thick. The temperature of the outside air is 25oC and that on the inside air is -20oC. The film co-efficient for outside air and brick is 45.4 W/m2K and for inside air and cement is 17 W/m2K. Find the heat flow rate and thermal resistance. Take kbrick = 3.45 W/mK, kcork = 0.043 W/mK, kcement = 0.294 W/mK

(OR)

19.	A 30 cm long glass is hung vertically in the air at 27oC, while its temperature is maintained at 77oC. Calculate the boundary layer thickness at the trailing edge of the plate. If the similar plate is placed in a wind tunnel and air is blown over it at a velocity of 4 m/s. Estimate the boundary layer thickness at the trailing edge of the plate.

20.	Air flows steadily at the rate of 0.5 kg/s through an air compressor, entering at 7 m/s velocity,             100 kPa pressure, and 0.95 m3/kg volume, and leaving at 5 m/s, 700 kPa, and 0.19 m3/kg volume. The internal energy of air leaving is 90 kJ/kg greater than that of the entering. Cooling water in the compressor jackets absorbs heat from the air at the rate of 58 kW. Compute the rate of shaft work input to the air in kW.

(OR)

21.	Derive the Steady Flow Energy Equation for a single stream of fluid entering and leaving the

control volume.

22.	Dry saturated steam at a pressure of 2.45 bar condenses on the surface of a vertical tube of height 1 m. The tube surface temperature is kept at 117oC. Estimate the thickness of the condensate film and the local heat transfer co-efficient at a distance of 0.2 m from the upper end of the tube.

(OR)

23.	Briefly explain the pool boiling curve for water with neat diagram.

24.	In a double pipe, counter flow heat exchanger 10000 kg/hr of oil having a specific heat of               2095 J/kg K is cooled from 80oC to 50oC by 8000 kg/hr of water entering at 25oC. Determine the heat exchanger area for an overall heat transfer co-efficient of 300 W/m2K. Take Cp for water as 4180 J/kgK.

(OR)

25.	A parallel flow heat exchanger has hot and cold water stream running through it, the flow rate are 10 and 25 kg/min respectively. Inlet temperatures are 75oC and 25oC on hot and cold sides. The exit temperature on the hot side should not exceed 50oC. Assume hi = ho = 600 W/m2K. Calculate the area of heat exchanger using NTU approach