Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	UNIT OPERATIONS OF CHEMICAL ENGINEERING		Time : 3 hours

Subject Code:	CL212			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name the different system of units.

2.	Give the formula for Trapezoidal rule.

3.	Give the equation for the overall material balance.

4.	Define absolute molal humidity.

5.	Define power number in agitation.

6.	Baffles in agitated vessels prevent ___________.

7.	What is free settling?

8.	The force in the direction of flow exerted by the fluid on the solid is called ____.

9.	Give examples of filter media.

10	Name some batch filters.

PART – B  (5 x 3 = 15 MARKS)

11.	Linearize the following equations:

y=a+bxy=aebxy=axny=a+cxny=xa+bxy=ax+bx2

12.	Discuss the various steps to be followed in material balance calculations.

13.	Discuss about the compressibility of filter cakes.

14.	How can you prevent swirling in agitated vessels? Explain.

15.	Explain the principle of pneumatic conveying.

PART – C  (5 x 15 = 75 MARKS)

16.	The heat transfer coefficient h is related to the velocity u of a fluid flowing through a pipe by h=aub

Determine the values of a and b from the following data:

| u, m/s | 0.3050 | 0.914 | 1.524 | 2.134 | 2.743 |
| --- | --- | --- | --- | --- | --- |
| h, W/m2K | 852 | 2100 | 3208 | 4258 | 5224 |

(OR)

[P.T.O]

17.	Evaluate 01e−x2dx dividing the interval into six equal parts, using trapezoidal rule, Simpson’s 1/3rd and 3/8th rule.

18.	a.	In the Deacon process of manufacture of chlorine, a dry mixture of hydrochloric acid gas 	and air is passed over a heated catalyst which promotes oxidation of the acid. Air is used in 	30% excess of that theoretically required.

4HCl+O2→2Cl2+2H2O

(i)	Calculate the weight of air supplied per kg of acid.

(ii)	Calculate the composition by weight of the gas entering the reaction chamber.

(iii)	Assuming that 60% of the acid is oxidized in the process, calculate the composition 		by weight of the gases leaving the chamber. 							(3+3+3)

b.	Briefly explain about the psychrometric chart with a neat schematic diagram.		(6)

(OR)

19.	SO2 reacts with pure O2 to form SO3. If the reaction is carried out with 100% excess oxygen as necessary for complete oxidation, but under such low temperature and pressure that the reaction goes only 60% to completion, calculate:

(i)	the mole fraction of SO2, O2, SO3 in the reactants and products.

(ii)	the weight fraction of the three gases in reactants and products.

(iii)	the average molecular weights of reactants and products

(iv)	the partial pressure of the products if the total pressure is 2 atm.

(v)	If the products were cooled to 20ºC and the pressure reduced to 720 mm Hg, what volume of gas would be obtained from 100 kg of SO2. 					 (5x3)

20.	On the assumption that the time required for mixing in a stirred tank is a function of the following variables: 	tm = f (μ, ρ, N, g, D, DT, W, H)

Obtain the dimensionless groups which are important in calculating time for mixing for geometrically similar arrangements.

(OR)

21.	a.	Explain with neat sketches, the different mixers for mixing liquids. 				 (7)

b.	A flat turbine with six blades is installed in a tank. The diameter of the tank is 1.83 m.         Da / Dt  is 0.33. The tank has 4 baffles. The turbine is operated at 90 rpm. The liquid has a viscosity of 10 cP and a density of 929 kg / m3. For the standard design, the following data is available:

| Rem | 5 | 100 | 1000 | 51000 | 500000 |
| --- | --- | --- | --- | --- | --- |
| Np | 14 | 4 | 4.1 | 5 | 5 |

i.	Calculate the power required in kW.											(4)

ii.	For the same conditions, except the viscosity of the solution is 100000 cP,

calculate the power required in kW.												(4)

22.	a.	What is criterion for a packed bed to be fluidized? Derive an expression for minimum 			fluidization velocity. 															(2+4)

b. 	Solid particles having size of 0.12 mm, a shape factor of 0.88 and a density of 1000 kg / m3, are to be fluidized using air at 2 atm pressure and 25ºC. The voidage at minimum fluidizing conditions is 0.42. Viscosity of air is 1.845X10-5 Pa.s

i. 	If the cross section of the empty bed is 0.3 m2, and the bed contains 300 kg of solid, calculate the minimum height of fluidized bed.										(3)

ii.	Calculate the pressure drop at minimum fluidizing conditions.   						(3)

iii.	Calculate the minimum velocity for fluidization.										(3)

(OR)

23.	Derive an expression for terminal settling velocity of a spherical particle. Deduce the relations in laminar flow and turbulent flow.

24.	a. Data for the filtration of CaCO3 slurry in water at 25ºC are reported as follows: at a constant   	   pressure of 194.4 kN/m2. The area of the plate and frame filter press was 0.0439 m2. The                      	  slurry concentration was 23.47 kg/m3 filtrate. Calculate the specific cake resistance and the 	      	  filter medium resistance.

| V X 103, m3 | 0.5 | 1 | 1.5 | 2.0 | 2.5 | 3.0 | 3.5 | 4.0 | 4.5 | 5.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time, s | 6.3 | 14.0 | 24.2 | 37.0 | 51.7 | 69.0 | 88.8 | 110 | 134 | 160 |

(3.5 + 3.5)

b.	 Classify the types of filtration equipment and explain the operation of a plate and frame filter press with the help of a neat sketch.										(3+ 5)

(OR)

25.	a.	Explain in detail about DORR THICKENER. 										(7)

b.	A slurry containing 5 kg of water/kg of solids is to be thickened to a sludge containing 1.5

kg of water/kg of solids in a continuous operation. Laboratory tests using five different concentrations of the slurry yielded the following data:

| Concentration (kgwater/kg solid) | 5.0 | 4.2 | 3.7 | 3.1 | 2.5 |
| --- | --- | --- | --- | --- | --- |
| Rate of sedimentation (mm/s) | 0.20 | 0.12 | 0.094 | 0.070 | 0.050 |

Calculate the minimum area of a thickener required to effect the separation of a flow of 1.33 kg/s of solids.																			(8)