Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	FUNDAMENTALS OF THERMAL FLUID SCIENCES FOR FOOD ENGINEERS										               Time : 3 hours

Subject Code: 	CL215			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	State the units of dynamic viscosity and kinematic viscosity.

2. 	Define turbulent flow.

3. 	Define Zeroth law of thermodynamics.

4. 	What is degree of superheat?

5. 	Convection is possible only in the presence of a -------- medium.

6. 	Any body is said to be in an unsteady or transient state, if the -------- varies with time.

7. 	The flow velocity in free convection is much ---------- than that encountered in forced convection.

8. 	What is thermal radiation?

9. 	Internal forced convection boiling is commonly referred to as --------- phase flow.

10. 	What are the two separate processes of nucleate boiling?

PART – B  (5 x 3 = 15 MARKS)

11. 	What is a venturimeter? Write the expression for calculating actual discharge of fluid flowing through venturimeter.

12. 	What is heat transfer? What are its positive and negative directions?

13. 	What do you understand by a term ‘Overall heat transfer co-efficient’?

14. 	Explain briefly the significance of Reynolds number for the flow over a flat plate.

15. 	Explain briefly the two modes of condensation.

PART – C  (5 x 15 = 75 MARKS)

16. 	a. 	Derive Bernoulli’s equation by integration of Euler’s equation. State the   assumptions made

for such derivation.															(5)

b. 	The water is flowing through a pipe having diameters 20 cm and 10 cm at sections 1 and 2

respectively. The rate of flow through pipe is 35 litres/sec. The section 2 is 6 m above the datum and the section 2 is 4 m above the datum. If the pressure at section 1 is 39.24 N/cm2, find the intensity of pressure at section 2.										(10)

(OR)

17.	a. 	Explain the principle of orifice meter with a neat sketch.							(5)

b. 	What is pitot-tube? How will you determine the velocity at any point with the help of pitot-

tube?																		(5)

c. 	Find the velocity of the flow of an oil through a pipe, when the difference of mercury level

in a differential U –tube manometer connected to the two tapings of the pitot-tube is 10 cm. Take co-efficient of pitot-tube 0.98 and specific gravity of oil 0.8.					(5)

[P.T.O]

18.	a.	Explain what do you understand by thermodynamic equilibrium? Explain mechanical,

chemical and thermal equilibrium.												(8)

b. 	State the limitations of first law of thermodynamics and give the statements of

second law of thermodynamics.												(7)

(OR)

19.	a.	Explain the following terms relating to steam formation:							(10)

(i) Sensible heat of water, (ii) Latent heat of vaporization (iii) Dryness fraction of

steam, (iv) Enthalpy of wet steam and (v) Superheated steam

b. 	A turbine, operating under steady-flow conditions, receives 4500 kg of steam per hour. The

steam enters the turbine at a velocity of 2800 m/min, an elevation of 5.5 m and a specific enthalpy of 2800 kJ/kg. It leaves the turbine at a velocity of 5600 m/min, an elevation of    1.5 m and a specific enthalpy of 2300 kJ/kg. Heat losses from the turbine to the surroundings amount to 16000 kJ/hr. Determine the power output of the turbine. 					(5)

20.	a.	Describe the three modes by which heat can be transferred from one place to another.	 (10)

b. 	Calculate the rate of heat loss for a red brick wall of length 5m, height 4 m and thickness

0.25 m. The temperature of the inner surface is 110oC and that of the outer surface is 40oC. The thermal conductivity of red brick, k = 0.70 W/m K. Also calculate the temperature at an interior point of the wall, 20 cm distant from the inner wall.						(5)

(OR)

21. 	Discuss in detail about transient heat transfer and lumped capacity analysis.

22. 	a. 	Explain the development of velocity boundary layer for laminar flow in a tube. 		(5)

b. 	Air at 20oC and 1 atm flows over a flat plate at 40 m/s. The plate is 80 cm long and is

maintained at 60oC. Assuming unit depth in z-direction, calculate the heat transfer rate from the plate.  The properties of air at 40o C are Pr = 0.7, cp = 1.007 kJ/kg K,

 = 1.906 x 10 – 5 kg/m-s and k = 0.02723 W/m K.								(10)

(OR)

23.	a.	Explain the terms ‘Absorptivity, Reflectivity and Transmissivity’.					(5)

b. 	Water at 50oC enters a 1.5 cm diameter and 3 m long tube with a velocity of 1 m/s. The tube

wall is maintained at a constant temperature of 90oC. Calculate the heat transfer co-efficient and the total amount of heat transferred if the exit water temperature is 64oC. The properties of water at the average bulk temperature are:  = 990 x 103 kg/m3, cp = 4184 J/kg K,             v = 0.517 x 10 – 6 m2/s, Pr = 3.15 and k = 0.65 W/m K.							(10)

24.	a.	Write a brief note on pool boiling and flow boiling.								(5)

b. 	A vertical tube of 50 mm outside diameter and 2 m long is exposed to steam at atmospheric

pressure. The outer surface of the tube is maintained at a temperature of 84oC by circulating cold water through the tube. Determine the rate of heat transfer and also the condensate mass flow rate. The properties of saturated vapour at Tsat = 100oC are: v = 0.596 kg/m3 and hfg = 2257 kJ/kg.  The properties of saturated liquid at Tf = 92oC are: l = 963.4 kg/m3 and

l = 306 x 10 – 6 N-s/m2 and kl = 677 x 10 – 3 W/m K. 								(10)

(OR)

25.	a.	State the assumptions made in deriving Nusselt’s equation for film condensation. 	(5)

b. 	Explain the flow regimes in two-phase flow through a tube.						(10)