Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	BIOCHEMICAL ENGINEERING					              Time : 3 hours

Subject Code: 	CL301			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Discuss the different types of agitators commonly used in fermenters. How to calculate power consumption in gassed and ungassed system?

(OR)

2. 	Explain the different types of valves used in fermenter.

Depict the importance of flow measuring devices in fermenter system.

3. 	A medium containing a vitamin is to be sterilized. Assume that the number of spores initially present is 105 /l. The values of the pre- Arrhenius constant and E0d   for the spores are                 E0d  = 65 kcal/g-mol     α = 1036  min-1. .For the inactivation of the vitamin, the values of           E0d  = 10 kcal/g-mol     α = 104 min-1 .The initial concentration of the vitamin is 30 mg/l. Compare the amount of active vitamin in the sterilized medium for 10-l and 10000- l fermenters when both are sterilized at 121º C. When we require in both cases that the probability of an unsuccessful fermentation be 0.001.Ignore the effects of heat up and cool down values.

(OR)

4. 	a.	Discuss batch and Continuous sterilization.

b.	List out the important factors to be considered in media formulation.

5.	Compulsory:

Pseudomonas putida with m =0. 5 h-1  is cultivated in a continuous culture under aerobic conditions where D=0.28  h-1 . The carbon and energy source in the feed is lactose with a concentration of So =2g/l. The effluent lactose concentration is desired to be S=0.1 g/l. if the growth rate is limited by oxygen transfer, by using the following information;

YX/S = 0.45gX/ gS,    YX/o2 = 0.25gX/ gO2  C*=8 mg/l

Determine the steady state biomass concentration (X)  and the specific rate of oxygen consumption ( qO2 )

6. 	a.	Results from a static gassing out method is given,

| Time (min) | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| % DO | 0 | 21 | 43 | 60 | 74 | 85 | 90 |

Calculate volumetric mass transfer coefficient for the reactor.

b.	What are the various resistances for O2 transfer from the source to the cellular reaction site? Derive the expression for OTR & OUR

(OR)

[P.T.O]

7.   	A fermenter used for antibiotic production must be kept at 27C. After considering the oxygen demand of the organism and the heat dissipation from the stirrer, the maximum heat transfer rate required is estimated as 600 kW. Cooling water is available at 10C ; the exit temperature of the cooling water is calculated as 25C. The heat transfer coefficient for the fermentation broth and cooling water are 2050 and 14000 Wm-2C-1 . It is proposed to install a helical cooling coil inside the fermenter; the outer diameter of the coil pipe is 8cm, the pipe thickness is 5mm and the thermal conductivity of the steel is 65 Wm-2C-1. An average internal fouling factor of             8000 Wm-2C-1 is expected; the fermenter side surface of the coil is kept relatively clean.What length of cooling coil is required?

8. 	A Stirred tank reactor is to be scaled down from 10m3 to 0.1m3 . The dimensions of the large tank are  Dt =2 m ; Di =0.5 m; N=100 rpm; H=3 Dt

a.	Determine the dimensions of the small tank (Dt , DI , H) by  using geometric similarity.

b. 	What would be the required rotational speed of the impeller in the tank if the following

criteria were used ?

i. Constant tip speed      ii. Constant  impeller Re number

(OR)

9.	a.	Compare and Contrast the Bubble column and Airlift Bioreactor.

b.	Discuss in detail about the various types of bioreactors for waste water treatment.