Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ADVANCED CHEMICAL ENGINEERING			              Time : 3 hours

Subject Code: 	CL303			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Explain in detail the classification of membrane processes. 						(8)

b.	Describe the types of equipment for gas permeation membrane processes.			(12)

2.	a.	Calculate the specific volume of superheated steam at 10 MPa a and 623 K using the ideal

gas law and the Van der Waals equation. The critical pressure and temperature of water are 22.076 MPa  and 647.11 K respectively. 										(12)

b.	A sample of caustic soda flakes contains 74.6% Na2O (by weight). Find the purity of flakes in percentage. 																(8)

(OR)

3.	a.	It is required to make 1000 kg mixed acid containing 60% sulphuric acid, 32% nitric acid

and the remaining water by blending three streams. Stream I contains 11.3% nitric acid, 44.4% sulphuric acid and the remaining water, stream II contains aqueous 90% nitric acid and stream III contains aqueous 98% sulphuric acid. All the percentages are by weight. Calculate the quantities of each of the three acids required for blending. 			(12)

b.	Calculate enthalpy of equimolar liquid mixture of n – pentane and n – hexane at

101.325 kPa. The bubble and dew points of the mixture are 321.6 K and 330 k respectively. The following table gives constants of the polynomial equation for liquid heat capacity: State the assumptions suitably made. 											(8)

| Compound | Constants |  |  |  |
| --- | --- | --- | --- | --- |
|  | a | B x 103 | c x 106 | d x 109 |
| N – pentane | 65.4961 | 628.628 | -1898.8 | 3186.51 |
| n – hexane | 31.421 | 976.058 | -2353.68 | 3092.73 |

4.	a.	Explain in detail the relation between Newton’s law and viscosity. 					(8)

b.	Derive an expression for Hagen – Poiseuille’s equation starting from basic principles.(12)

(OR)

5.	a.	Explain in detail the gas moving machineries. 									(8)

b.	Describe the construction and working of centrifugal pumps. 						(12)

6.	a.	Explain in detail the basic mechanisms of heat transfer. 							(16)

b.	Calculate the heat loss per unit area of surface for an insulated wall composed of 25.4  mm thick fiber insulating board, where the inside and outside temperatures are 353 K and 297 K respectively. Thermal conductivity of board = 0.048 W/m.K .						(4)

(OR)

7.	a.	Derive an expression for effectiveness of parallel flow and counter flow heat exchangers. (8)

b.	Describe the construction and working of shell and tube heat exchanger. 			(12)

[P.T.O]

8.	a.	Explain in detail the basic theory of filtration. 									(12)

b.	Data for the laboratory filtration of CaCO3 slurry in water at 299 K are reported as follows at

a constant pressure of 338 kN/sq. m. The filter area of the plate and frame filter press was

A = 0.0439 sq. m. and the slurry concentration was 23.47 kg/cu. m. calculate the constants Kp and B from the experimental data given. 										(8)

| Time (s) | 4.4 | 9.5 | 16.3 | 24.6 | 34.7 | 46.1 | 59.0 | 73.6 | 89.4 | 107.3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Filtrate volume (l) | 0.50 | 1.00 | 1.50 | 2.00 | 2.50 | 3.00 | 3.50 | 4.00 | 4.50 | 5.00 |

(OR)

9.	a.	Describe the types of equipment for agitation. 									(12)

b.	Explain in detail the mixers for powders and pastes. 								(8)