Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	OPTICAL FIBER COMMUNICATION				       	       Time : 3 hours

Subject Code: 	CS363			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Give the mode theory for circular waveguides.

2.	Explain the following terms a) Group delay b) Material Dispersion c)Waveguide dispersion d)Polarisation mode dispersion e) Intermodal distortion.

(OR)

3.	With neat diagram explain the construction and working of high radiance surface emitting LED. Derive quantum efficiency and LED power.

4. 	a.	Assume a simple model of photo detector receiver and give the noise performance on

receiver in terms of SNR.														(10)

b.	An InGaAs pin photodiode has the parameters at a wavelength of 1300nm

ID = 4nA, η= 0.90, RL = 1000Ω, and the surface leakage current is negligible. The incident optical power is 300 nW  (-35 dBm), and the receiver bandwidth is 20MHz.Find the various noise terms of the receiver. 												(10)

(OR)

5.	Discuss about the various types of receiver pre amplifiers.

6. 	What are the noise effects of digital Receiver system performance?

(OR)

7. 	a.	Explain the operational principles of WDM.										(10)

b.	Explain the 2x 2 waveguide coupler and star couplers				         			(10)

8.	a.	What are the general applications of optical amplifiers? Explain its types. 			(10)

b.	Explain in detail about optical CDMA.											(10)

(OR)

9. 	Explain in detail the broadcast and select WDM networks and wavelength routed networks.