Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MICROWAVE ENGINEERING						              Time : 3 hours

Subject Code: 	CS375			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	State and prove poynting theorem.				  	   	                 				(10)

b.	A uniform plane wave of frequency 100 kHz is incident on the plane interface between free space (z<0) and sea water (z>0, σ=4 S/m, єr=80 and µ=µ0). Determine a) the amplitude of the incident wave electric field for which the amplitude of the transmitted wave electric field at z=1m is 1mV/m and b) the depth at which the transmitted wave electric field is 1/e times the incident wave electric field.                                              						(10)

2.	Describe in detail about the planar transmission lines and specify the losses, advantages and disadvantages of each planar transmission line.

(OR)

3.	Describe in brief about the different types of broadband impedance matching techniques.

4.	a.	Explain the properties of S-parameters for junction of ports having common characteristic

impedance.							                                        					(10)

b.	Compare [s], [z] and [y] matrices. 	  						    					(10)

(OR)

5.	Explain with neat diagrams the working of 											(10+10)

a.	precision rotary phase shifter        b.	 MIC phase shifters.

6.	Design a microstrip circular disk resonator operating in the TM110 mode at 2GHz with substrate of relative dielectric constant 6-j0.005 and is 1/16’’ thick. The disk is made of copper. Determine the required radius of the disk and Q of the resonator.

(OR)

7.	Describe in detail about the different types of coupled cavities which are used in microwave filter circuits and wavemeters.

8.	a.	Explain the prototype low pass design by Insertion loss method.                                  	(15)

b.	Design a low pass 50 ohm chebyshev coaxial line filter with 0.1 dB pass band ripple and number of sections=3. The band edge freq is 2 GHz.[g1=1.0315, g2=1.1474,g3=1.0315, g4=1.000 = g0]                                                                            						(5)

(OR)

9.	Find the even and odd mode impedance of a 6-section parallel coupled chebyshev microstrip band pass filter at 4 GHz with a 100MHz bandwidth. The parameters are єr=9.8 and h=0.5mm. The input and output impedances are 50ohms.[ g0=1, g1=1.1681, g2=1.4039,g3=2.0562, g4=1.517 , g5=1.9029, g6=0.8618,g7=1. 3554]