Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	SOLID STATE CIRCUITS – I 								Time : 3 hours

Subject Code:	EC205			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is Voltage Regulator?

2.	Mention the disadvantages of π filter?

3.	What is the value of stability factor of common base circuit?

4.	What types of biasing is used in FET?

5.	What is cascade amplifier?

6.	What is the necessity of class D amplifier?

7.	What are the types of feedback?

8.	What is the sensitivity of an amplifier?

9.	What is the function of tuned voltage amplifier?

10.	Why three RC networks are needed for a phase shift oscillator?

PART – B  (5 x 3 = 15 MARKS)

11.	A half wave rectifier is used to supply 30V dc to a resistive load of 600 ohm. The diode has a forward resistance of 25 ohm. Find the maximum value of ac voltage required at the input.

12.	Explain any one bias compensation techniques used in transistor biasing.

13.	Differentiate between voltage amplifier and power amplifier.

14.	A negative feedback is used to reduce the noise from an amplifier by 80%

i.	What must be the percentage of negative feedback to accomplish this, if the input voltage gain is 100?

ii.	What will be the voltage gain with feedback?

15.	What is frequency stability? Mention the factors which contribute to the change in frequency.

PART – C  (5 x 15 = 75 MARKS)

16.	Draw the circuit diagram of full wave rectifier with center tap connection. Explain its working with circuit diagram. Explain about voltage regulators.

(OR)

17.	With the help of schematic diagram explain the working of π filter and full wave rectifier and derive the expression for the ripple factor.

18.	Draw a voltage divider bias circuit and derive an expression for its stability factor.

(OR)

19.	 Explain about the self bias circuit and fixed bias circuit and fixed bias circuit of FET? Derive its stability factors.

20.	a.	With a neat circuit diagram explain the principle of operation of a single ended class A

power amplifier.

b.	Write a note on wideband amplifiers and explain its working?

(OR)

21.	Draw the circuit diagram of a two stage RC coupled CE transistor and explain its working? Show 	how the magnitude and the phase of its voltage gain vary with frequency.

22.	Give a circuit for differential amplifier? How the drift is reduced in such type of amplifier.

(OR)

23.	Explain the effect of voltage series negative feedback on BJT amplifier in terms of the parameters namely input impedance, output resistance, bandwidth, distortion, noise and gain stability.

24.	Describe the construction of phase shift oscillator and explain its working. 	Derive the expression for the frequency of oscillation and the minimum gain required for sustained oscillation

(OR)

25.	Describe the operations of a double tuned voltage amplifier and explain how its frequency response is better than that of a single tuned voltage amplifier.