Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ELECTROMAGNETIC FIELDS								Time : 3 hours

Subject Code:	EC233				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention one property of dot product

2.	What is electric flux

3.	What is magnetic lines of force

4.	Define Magnetic moment

5.	Poisson’s equation is ___________

6.	State the relation between I and J

7.	Classify Magnetic materials

8.	If N is the number of turns and l is the length then inductance of a solenoid L  is _________H.

9.	According to vector identity, divergence of curl of vector is ________

10.	Define Faraday’s law

PART – B  (5 x 3 = 15 MARKS)

11.	Mention few applications of Gauss law

12.	List the applications of Ampere’s circuital law

13.	Derive the integral form of continuity equation of current.

14.	Define Magnetization and permeability

15.	What is the equation of continuity of current in point or differential form

PART – C  (5 x 15 = 75 MARKS)

16.	Find the total charge inside a volume having volume charge density as 10z2 e-0.1xsin πy C /m3. The volume is defined between -2≤x≤2, 0≤y≤1, 3≤z≤4.

(OR)

17.	A particular vector field F = r 2 cos2φ a r + z sin φ a φ is in cylindrical system. Find the flux emanating due to this field from closed surface of the cylinder 0≤z≤1, r=4. Verify divergence theorem.

18.	Define Biot-Savart Law and explain how it can be applied to distributed forces

(OR)

19.	A magnetic circuit employs an air core toroid with 500 turns, cross sectional area 6cm2 , mean radius 15cm and 4A coil current. Determine reluctance of the circuit, flux density and magnetic filed intensity

20.	Determine whether or not following potential fields satisfy the Laplace’s equation

a.	V=x2 –y2+z2  	b.	V=rcosφ +z    	c.	V=rcos θ +φ

(OR)

[P.T.O]

21.	A parallel plate capacitor with a separation of 1cm has 29kV applied, when air was the dielectric used. Assume that the dielectric strength of air as 30 kV/cm. A thin piece of glass with εr =6.5 with a dielectric strength of 290 kV/cm with thickness 0.2 cm is inserted. Find whether glass will break or air

22.	Explain and derive the boundary conditions for a conductor free space interface

(OR)

23.	A air core toroid has a mean radius of 40mm and is wound with 4000 turns of wire. The circular cross section of the toroid has a radius of 4mm. A current of 10A is passed in the wire. Find the inductance and the energy stored.

24.	Derive Maxwell’s equations from Ampere’s circuit Law.

(OR)

25.	Derive the point and integral form of Poynting theorem.