Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MICROPORCESSORS AND APPLICATIONS				Time : 3 hours

Subject Code:	EC238				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name the interrupt pins available in 8085?

2.	Calculate the time needed to execute an instruction ADD B, if the clock

frequency is 2Mhz.

3.	Define pipelining

4.	Calculate the Physical address if SS=4200,IP=1278,CS=3680

5.	What are the errors occur during data transmission

6.	Write the control word in 8255,when all the ports are connected to display

7.	Which mode of 8253 is used for generating square wave

8.	If two 8259’s are cascaded ,how many I/O devices can be connected

9.	How many address lines are needed to access a 32K memory

10.	Name any two interfacing devices used when microprocessor is connected to

high power devices

PART – B  (5 x 3 = 15 MARKS)

11.	Write an Assembly language program in 8085 to find two’s complement of a number. The

number is in location 5000 and store the result in 5001

12.	What are the advantages of memory segmentation

13. 	Explain the following signals with the direction of flow

STB, IBF, OBF

14.	What are the features of 8275 CRT controller

15.	Compare Absolute decoding and partial decoding

PART – C  (5 x 15 = 75 MARKS)

16.  Draw the architecture of 8085 and explain the functions of each block

(OR)

17.	a.	Draw the timing diagram of the instruction  2050  STA 4689 .Opcode of STA is 32        (8)         	b.	Write an ALP in 8085 to transfer the block of ten data starting from the location 4000 to

5000   																		     (7)

18.	a.	Explain the addressing modes of 8086 with example            						     (10)

b.	Explain the following instruction with example                                                                   (5)

MUL AB

REP MOVSB

(OR)

19. 	Explain 8086 processor in minimum mode with necessary diagrams

[P.T.O]

20.	With the help of a block diagram, control and status words explain  Asynchronous serial data communication interface 8251

(OR)

21.	a.	Write the sequences of operations carried out during DMA? Explain the

different modes of data transfer                              									(9)

b.	Write the control word format of 8255 in I/O mode and BSR mode . Write the control word to reset portC pin3               													       (6)

22.	Explain the stepwise response of 8259 programmable interrupt controller

with ICW’s and OCW’s

(OR)

23.	With a neat block diagram ,Explain the programmable interval timer 8253 with

any four  modes of operation

24.	Design a memory system for 8085 processor such that it should contain two 8kbyte EPROM and

two 4Kbyte RAM.       Also write the address mapping.

(OR)

25.	Explain with block diagram and flow chart ,  how a microprocessor is used to

control the speed  of the DC motor .