Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MICROCONTROLLERS AND APPLICATIONS				Time : 3 hours

Subject Code:	EC239			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In the 8051, the program counter is ______ bit wide.

2.	How does the CPU know where to return to, after executing the RET instruction?

3.	Which of the 8051 ports need pull up resistors to function as an I/O port?

4.	What is the main difference between 8031, 8051 and 8751?

5.	What is the hand shaked port?

6.	What is the use of input capture register in 68HC11?

7.	When the synchronous communication is used?

8.	What is the relation between watchdog timer and E clock?

9.	What are the reasons for using SCL and SDA pins in PIC family?

10.	What is the operation of the given instruction?

Instruction - BCF port B, 0

PART – B  (5 x 3 = 15 MARKS)

11.	Briefly explain the program status word register in 8051?

12.	Give elaborate about the programming steps to be taken in the 8051 to receive the character bytes serially?

13.	Mention the functions of various port pins in Motorola 68HC11 at expanded multiplexed mode of operation.

14.	Explain the controlling registers for the various functions in Motorola     68HC11.

15.	Explain the initialization procedure for the CCPI compare mode in PIC microcontroller.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the memory structure of 8051 with necessary diagrams.

(OR)

17.	Explain the different addressing modes of 8051 with the help of suitable examples.

18.	Describe in detail about the various operating modes of the timer / counters and associated control registers in 8051.

(OR)

19.	Explain in detail about the different types of interrupts in 8051 with program addresses and the interrupt control registers.

20.	Discuss about the types of counters and features of each and every counters.

(OR)

21.	Explain the functions of input capture 3 timer, 2 timer in 68 HC11 micro controller.

22.	Explain in detail about the features of SPI in MC68HC11.

(OR)

23.	a.	Explain the system for programming the watchdog timer interrupts.

b.	Explain the A/D conversion features of MC68HC11.

24.	a.	Explain the controlling mechanism of DC motor for direction and  speed using any one of

the microcontrollers.

b.	Discuss the operation involved in I2C bus interface.

(OR)

25.	List and explain the different group of instructions in PIC microcontroller.