Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	COMMUNICATION THEROY & SYSTEMS				Time : 3 hours

Subject Code:	EC242				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define Communication.

2.	What is meant by wireless communication?

3.	Define modulation index of AM.

4.	Draw the spectrum of AM.

5.	What is meant by high level modulation?

6.	Define Demodulation

7.	Define heterodyning.

8.	Define fidelity of the radio receiver.

9.	Define noise.

10.	Define figure of merit.

PART – B  (5 x 3 = 15 MARKS)

11.	List different types of modulation,

12.	Compare AM and  FM

13.	Derive the relation between FM and PM.

14.	When superhetrodyne receiver is tuned to 555kHz,if local oscillator provides the mixer with an input at 1010 kHz.  What is the image frequency?

15.	Consider the resistor of 20 k and 50 k are at the room temperature (290k) for a bandwidth of 100 kHz.  Calculate the thermal noise voltage generated by each receiver.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the block diagram of communication system.

(OR)

17.	Discuss the advantages and need for modulation in detail.

18.	Derive the power relations in AM also derive transmission efficiency.

(OR)

19.	Derive the mathematical expression for FM signal and draw the frequency spectrum of FM Wave.

20.	Draw the block diagram of high level and low level modulation of AM system and explain its operation.

(OR)

21.	a.	In a FM system, the modulating frequency is 600Hz and modulating voltage is 2.4 v and

modulation index is 60.  Calculate the max deviation.  What will be the modulation index when modulation frequency is 250Hz and modulation voltage is 3v.				 		 (10)

b.	Compare FM over AM. What are the applications of FM signal?					   (5)

22.	With neat diagram explain the function of super heterodyne receiver.

(OR)

23.	Draw the block diagram of Armstrong FM transmitter system and explain its functions.

24.	Explain various sources of noise.

(OR)

25.	Derive the expression for signal to noise power ratio for SSB-SC AM system.

.