Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	TRANSMISSION LINES AND ANTENNA SYSTEMS		Time : 3 hours

Subject Code:	EC243				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Frequency distortion can be reduced by using ------------

2.	Phase constant when multiplied by the length of the transmission line is called ---- length of the line

3.	-------- are points of zero voltage or current on the standing waves

4.	Write the expression for the propagation constant of a zero dissipation line

5.	The dominant mode in a rectangular waveguide is ----

6.	A waveguide behaves as a ----- filter

7.	The power gain of a half wave dipole with respect an isotropic radiator is ---- dB

8.	The radiation and loss resistances of an antenna are 72 ohms and 8 ohms respectively. Determine the efficiency

9.	A half wave dipole at a frequency of 100 MHz has a length of ---- metres

10.	------ antenna is frequency independent

PART – B  (5 x 3 = 15 MARKS)

11.	Write the condition for a line to be distortionless. State whether such a line can be realized in practice

12.	Mention the advantages of short circuited stubs over open circuited stubs

13.	Write the expressions of  the characteristic impedances for the TE and TM waves between parallel lines

14.	Give the applications of reciprocity theorem

15.	Find the beam width between first nulls and power gain of a 2 m paraboloid reflector operating at 6000 MHz

PART – C  (5 x 15 = 75 MARKS)

16.	Derive the transmission line equations and obtain expressions for the voltage and current at any point on the transmission line

(OR)

17.	a.	An open wire transmission line has R = 10 ohms per km, L = 0.0037 Henry  per km,             		G = 0.4 x 10-6 mhos per km and C = 0.0083 x 10-6 Farad per km.   Determine the 				characteristic impedance and propagation constant.  Assume frequency as 1000 Hz        (8)

b.	Write a brief note on loading of transmission lines               						     (7)

18.	a.	Discuss the applications of Smith chart with suitable illustrations            			    	(8)

b.	Discuss with neat diagram the theory of single stub matching                  			    	(7)

(OR)

19.	a.	Derive the input impedance of open and short circuited dissipationless  line. Illustrate with

suitable diagrams the variation of the input impedance   as a function of length.                 (7)

b.	Give a brief account of power and impedance measurements on a transmission line           (8)

20.	Derive expressions for the electric and magnetic field components for the TM m,n waves in a rectangular waveguide.

(OR)

21.	a.	Draw and explain the different methods of excitation of modes in a waveguide                (7)

b.	Discuss the electric and magnetic filed components of TE waves in a circular waveguide with relevant expressions and diagrams                                            						(8)

22.	a.	Discuss the broadside array of n isotropic sources of equal amplitude and spacing with

necessary diagrams and expressions                                                                                    (8)

b.	Illustrate pattern multiplication with an example                                                                (7)

(OR)

23.	Explain the salient features and characteristics of sky wave propagation

24.	a.	Draw and explain the construction, action and radiation pattern of Yagi Uda antenna      (8)

b.	Write a brief note on lens antennas                                                                                     (7)

(OR)

25.	Discuss the following antennas with suitable diagrams and expressions

a.	Slot antenna                                                                                                                        (8)

b.	Rhombic antenna                                                                                                                (7)