Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	DIGITAL SIGNAL PROCESSING					               Time : 3 hours

Subject Code: 	EC245			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	State the condition for a system to be causal?

2.	What is the main advantage of FFT?

3.	Why FIR filter is always stable?

4.	Why is cascade form realization preferred in FIR filters?

5.	Write any one property of Butterworth low pass filters

6.	A causal and stable IIR filter cannot have linear phase. [State True or False]

7.	What are the different types of representation of numbers in digital system?

8.	What are the methods used to prevent overflow?

9.	Which operation is used to access many memory cycles?

10.	What is the use of instruction BANZ  in TMS 320C5416?

PART – B  (5 x 3 = 15 MARKS)

11.	Compute y(n)=x(n) * h(n) where x(n)=h(n)={1,2,-1}

12.	Distinguish between FIR and IIR filters.

13.	By impulse invariance method obtain the digital filter transfer function and the differential equation of analog filter H(s)=1/s+1.

14.	Determine dead band of the filter described by the difference equation y(n)=0.95y(n-1) + x(n).

15.	What are the different phases when executing MACD instruction?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Compute the DFT of x(n) = {1,1,-2,-2}.											(8)

b.	Find circular convolution of h(n) ={1,1,1} and  x(n) = {1,2,3,1}.					(7)

(OR)

17.	Find DFT of a sequence x(n) = {1,2,3,4,4,3,2,1} using DIT FFTalgorithm.

18.	Determine the filter coefficients for N =13 for an ideal low pass filter using a Hamming window with a frequency response.

Hd(ejw)=e−j α w ; for−π8≤w≤π8 0 ; forπ8≺|w|≤π

(OR)

19.	Determine the coefficients of a linear phase filter of length 13 which has a symmetric unit sample response and a frequency response that satisfies the condition

H(k) =1 ;k=0 to 2 0 ; k=3 to 5

[P.T.O]

20.	Design a digital Butterworth filter that satisfies the following constraints using bilinear transformation. Assume T=1 s.

0.9≤H(ejw)≤1 ; 0≤w≤π2 H(ejw)≤0.2 ; 3π4≤w≤π

(OR)

21.	Design a Chebyshev filter for the following specifications using impulse invariance method.

.

22	Derive the condition for steady state noise power due to input quantization and prove that

the SNR increases approximately by 6 dB for each bit added to register length.

(OR)

23.	Write notes on limit cycle oscillations.

24.	Explain briefly the Harvard architecture.

(OR)

25.	Explain in brief about LMS and RLS adaptive algorithms.