Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	DIGITAL COMMUNICATION						               Time : 3 hours

Subject Code: 	EC249			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the Nyquist rate for the signal x (t) =10 cos (40πt)?

2.	 ------------ and ----------- noises occur in Delta modulation.

3.	Write down the necessary condition for a minimum shift keying.

4.	Plot the BPSK signal for the given sequence 01001101.

5.	White noise has variance as _______.

6.	Give the impulse response of the matched filter.

7.	Define entropy?

8.	Determine the average information if there are 5 symbols with probability                            	12,14,18,14 and12.

9. 	State the Balance property.

10.	Find the processing gain of the spread spectrum system, if the number of flip flops used to generate the PN sequence is three.

PART – B  (5 x 3 = 15 MARKS)

11.	Find the number of bits per sample, when a signal m(t) = 5 sin(2πft) is quantized using uniform quantizer to achieve an SNR= 1536.

12. 	Compare any six features of BFSK and BPSK.

13. 	Draw the signal space diagram QPSK.

14. 	State the properties of syndrome decoding.

15. 	A PN sequence is generated using a feed back register of length m = 4. The chip rate is 107 chips per second. Find the length, chip duration and the period of the PN sequence.

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat diagram, explain the operation of DM. Compare its performance with PCM, DPCM and PAM.

(OR)

17. 	a.	What is ideal Nyquist channel? What are the merits and limitations?			  		(7)

b.	What is ISI? How it can be minimized? Explain with suitable methods.					(8)

18. 	a.	Draw the block diagram of binary FSK transmitter and explain its operation with signal

constellation diagram.									  							(8)

b.	Draw the block diagram of differentially encoded PSK system and explain its operation. 	(7)

(OR)

19. 	Draw the block diagram of MSK system and explain its principle and operation with signal constellation diagram.

[P.T.O]

20. 	Explain the basis of operation of a matched filter receiver with suitable diagrams. Derive

the condition for which the out signal to noise ratio is maximized.

(OR)

21. 	What is coherent reception? With necessary equations and signal space diagram, obtain   the probability of error for BPSK system.

22.	a.	Derive the capacity of a gaussian channel.											(8)

b.	Describe the syndrome method and standard array decoding method of linear

block codes.																		(7)

(OR)

23. 	a.	Explain the Shannon’s theorem?													(5)

b.	Decode the following received sequence 1 0 0 1 1 1 0 0 using ½ rate convolutional code whose g1 (x) = 1 + x + x2 and g2 (x) = 1 + x2.										             (10)

24. 	a.	State and explain the properties of maximum length sequences.			  			(5)

b.	Draw the block diagram of DS – spread spectrum system transmitter and receiver and explain the function performed by each block in brief. Derive the expression for jamming margin and processing gain.																	(10)

(OR)

25. 	Explain the principle of operation of slow frequency hopped M-ary FSK spread spectrum system with one example. Comment on its performance with respect to DS system.