Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	DIGITAL SYSTEM DESIGN USING VHDL			               Time : 3 hours

Subject Code: 	EC250				  						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	‘ASM’ stands for _________________.

2.	____________ and _______________are the two common types of symbols used

in ASM chart.

3.	In digital systems, the speciation of operation are often given in terms of high level operations describing transformations from one to other storage units or registers.   Such description are known as _________descriptions.

4. 	What is Emulation?

5. 	What is FPGA?

6. 	What is CPLD?

7. 	Write a VHDL Program for full adder.

8. 	What is the simulation action of the following VHDL statement?

clk <= not clk after 10 nsec,

9. 	Give example for conditional assignment statement in VHDL.

10. 	Mention the different types of Modeling in VHDL.

PART – B  (5 x 3 = 15 MARKS)

11. 	With neat diagrams compare programmed and un-programmed PALS.

12. 	Give the following RTL definitions in form of functions .Comment on the nature of operation or 	mention the descriptions of the operation.

a).   A  B

b).   C  A – B

c).   PC B if A =0,

13. 	Compare semi custom and full custom IC design.

14. 	Model a D-flip flop using a VHDL program.

15. 	Write a VHDL Program to add two 4 bit vectors and a carry and further return a five bit sum.

PART – C  (5 x 15 = 75 MARKS)

16. 	Using K-map technique obtain the Boolean function for the following and thus

realize the Boolean function using PLA.

F0= Σ m (0,1,4,6)

F1= Σ m (2,3,4,6,7)

F2= Σ m (0,1,2,6)

F3= Σ m (2,3,5,6,7)

(OR)

[P.T.O]

17.	Construct a PAL fuse map diagram for the following output functions .Comment how is this architecture differs from PLA architecture. In below functions  (‘ )   means  complement.

B0=A0

B1=1

B2=A0’A1

B3=A2’A1A0 + A2A1’A0

B4=A2’A1’ +A2’A0

B5=A2A1

18. 	Explain the design Process of a micro program controller in detail.

(OR)

19. 	Explain about the bit slice computer Architecture and its features.

20. 	Explain the layout and logic cell design of Xilinx 3000 series.

(OR)

21. 	Explain in detail about Xilinx 4000 series logic family.

22. 	With suitable examples discuss all the types of VHDL operators.

(OR)

23.	Write a VHDL Program for four bit full adder, using the structural description .Draw its basic structure showing the inputs and outputs signals.

24.	With suitable state transition graph design a traffic light controller and write the VHDL for the traffic light controller.

(OR)

25. 	With suitable state transition graph design a CPU controller.Also discuss the VHDL codes for ALU and register memory operations.