Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ANALOG ELECTRONICS									Time : 3 hours

Subject Code:	EC260				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The Peak Inverse Voltage across the diode in Half wave rectifier is________

2.	The ripple factor in inductor filter is approximately proportional to load resistance(True/False)

3.	The relation between α and β in BJT is _________

4.	A transistor acts as a current source (True/False)

5.	The operating cycle of Class A amplifier is _________

6.	If in class B power amplifier, VCC=20v RC=10Ω, maximum power is _________

7.	The CMRR of an ideal op-amp is __________

8.	The CMOS IC devices have lower power ratings than those of bipolar devices (True/False)

9.	Bistable multivibrator has how many states?

10.	Astable multivibrator does not require any external trigger pulse for the transition (True/False)

PART – B  (5 x 3 = 15 MARKS)

11.	What is a clamper circuit? How it adds a dc level to the output voltage

12.	Define intrinsic stand off ratio

13.	Define Barkhausen criterion.

14.	Define slew rate

15.	What is triggering of multivibrator

PART – C  (5 x 15 = 75 MARKS)

16.	A sinusoidal voltage whose Vm=24v is applied to half wave rectifier. The diode may be considered to be ideal and RL=1.8KΩ is connected as load. Find out Im , Irms, Idc an ripple factor

(OR)

17.	Design a π- section filter full wave circuit to obtain a ripple factor of 0.01 at a frequency of 50Hz

18.	Explain the construction operation and VI characteristics of Triac with their equivalent circuit

(OR)

19.	If Vp= -4V, ID=4mA and IDSS=6mA Find VGS for JFET

20.	Explain Wien-bridge oscillator and derive the expression for its frequency of oscillation

(21)

21.	In a RC phase shift oscillator, Rf = 0.5MΩ and Cf = 100pf. Find the frequency of oscillation if                        Rc =0.5MΩ.

22.	Explain how operational amplifier work as log and antilog amplifiers

(OR)

23.	What are the two external frequency compensation techniques? Explain them

24.	Explain in detail about instrumentation amplifier and compare their performance with the ordinary differential amplifier by deriving the equation for the output voltage.

(OR)

25.	With neat block diagram explain the internal block diagram of 555 timer.