Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ELECTRONIC INSTRUMENTATION						Time : 3 hours

Subject Code:	EC261			       						                	Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A pressure measuring instrument is calibrated between 10 bar and 250 bar. The scale span of the instrument is__________

2.	The units whose sizes cannot be chosen independently are called ________

3.	___________ has a negative temperature coefficient of resistance.

4.	___________ can act as in inverse transducer.

5.	Time required for the pulse to go from 10% to 90% of its amplitude is called ______

6.	Triangular waveform cab be obtained by integrating a ________

7.	An aquadag is used in a CRO to collect_________

8.	In a CRT, the _____ is located between pre-accelerating anode and accelerating anode.

9.	Digital Instruments have an input impedance of the order of_______

10.	In a digital frequency meter, the ___________ is used for converting sinusoidal waveforms into rectangular pulses.

PART – B  (5 x 3 = 15 MARKS)

11.	Define the terms static error and static correction.

12.	State Piezoelectric effect.

13.	List out the application of X-Y recorders.

14.	Pulses of frequency 1 MHz are applied for a time base selector of digital frequency meter which consists of 6 frequency dividers, each dividing the incoming frequency by a factor of 10. What is the time base setting at the output of 4th frequency divisor?

15.	Define Total Harmonic Distortion.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the various classifications of error with suitable examples.

(OR)

17.	Draw the circuit of a wheat-stone bridge and derive the conditions for balance. Also derive an expression for bridge sensitivity of a wheat-stone bridge with equal arms.

18.	Explain the working principle of a strain gauge and derive the strain resistivity relation.

(OR)

19.	Explain the construction and working principle of Linear Variable Differential Transformer. Also list out the advantages and applications of a LVDT.

20.	Explain the working of a Spectrum analyzer with a neat block diagram. Also list out its applications.

(OR)

21.	Discuss the working of a pulse generator with a neat circuit diagram.

[P.T.O]

22.	Explain the working principle of Digital Storage Oscilloscope with a neat block diagram.

(OR)

23.	Explain the working of a galvanometric type Strip Chart recorder with a neat block diagram. 	Also discuss the different types of tracing systems used in it?

24.	Explain the working principle of a ramp type and integrating type digital voltmeter with a neat block diagram.

(OR)

25.	a.	Write short notes on IEEE 488 standard.											(5)

b.	Discuss the working of a Digital Frequency meter with a neat block diagram.		(10)