Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	FOOD ENGINEERING - II							              Time : 3 hours

Subject Code: 	FP306			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

a.	Define Dalton’s law of partial pressure and Raoults law.

b.	Explain briefly about the Rotary Bed Adsorbing equipment with a neat sketch

2.	a.	Define Boiling point elevation and Duhring’s rule.

b.	Describe briefly about the Agitated type thin film evaporator with a neat sketch

(OR)

3.	Apple juice is being concentrated in a natural circulation single effect evaporator. At Steady state condition, dilute juice is the feed introduced at a rate of 0.67 kg/s. The concentration of the dilute solution is 11% total solids. The juice is concentrated to 75% total solids. The specific heats of dilute and concentrated apple juice are 3.9 and 2.3 KJ/kgºC respectively. The steam pressure is maintained at 304.42 kPa [Saturated steam temp = 134ºC and the latent heat of steam =     2231.49 kJ/kg]. The inlet feed temp is 43.3ºC. The product inside the evaporator boils at 62.2ºC [Latent heat of vapour = 2613.4 kJ/kg]. The over all heat transfer co-efficient (U) is                  943 W/m2 ºC. Assume negligible boiling point elevation. Calculate the mass flow rate of the concentrated product, steam requirement, the economy of the evaporator and the over all heat transfer area of the evaporator.

4.	a.	Define water activity and dehydration of Food products.

b.	Explain with schematic diagram about a Rotary drum drier for liquid and high viscous foods.

(OR)

5.	a.	Describe about the drying rate curves and the drying characteristics of food products

b.	Explain with neat sketch working of a Fluidized bed dryer

6.	Describe briefly about the contact equilibrium process for solid – liquid extraction with suitable examples.

(OR)

7.	a.	What are all the materials used in super critical fluid Extraction (SCFE) process as solvents.

b.	Explain briefly about a SCFE equipment with a neat sketch and a flow chart.

8. 	What are all the food freezing systems available and explain any one of them.

(OR)

9.	How the freezing time is predicated? Derive	Plank’s equation for estimating the freezing time.