Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	COMMUNICATION ENGINEERING				               Time : 3 hours

Subject Code: 	IT236			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention different types of noises

2.	A system has 150 khz bandwidth available for a 10 khz modulating signal. What is the appropriate deviation to be used?

3.	A 10 V differential signal is received with 5V of common mode voltage (CMV). After the receiving circuitry, this CMV is reduced to 0.01 V. What is CMRR.

4.	Unit of Slew rate is _______

5.	Parity bit is used to detect _______

6.	Define baud rate

7.	What is DTE

8.	What are the four functional groupings of RS 232 signal lines

9.	LORAN is ______

10.	What are the three segments of GPS system

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate mixer and local oscillator

12.	How large a dish diameter in wavelength and meter is needed for 60db gain at 300Mhz

13.	What do you mean by Frame Synchronization?

14.	How long does it take to dial the number 784-3742 a)using pulse dialing with 0.5s inter digit time b) with tone dialing where digit push buttons are held down for 0.25s and the inter digit time is also 0.25s?

15.	Calculate the power received when the transmitted signal is 1000W at 1Ghz (0.3 m), the target is 10000m away (about 5.7 miles), the target RCS is 1 m2, and the antenna gain is 40 dB?

PART – C  (5 x 15 = 75 MARKS)

16.	Discuss any two ways of generating an SSB signal? What are the advantage and disadvantage of each?

(OR)

17.	Explain the operation of PLL as a closed loop system. How does it produce a signal that tracks the input signal frequency and phase?

18.	Explain the operating principle of Yagi Antenna and mention its advantages

(OR)

19.	Calculate physical length corresponding to ¼ λ at 500 Mhz for a cable when the propagation velocity is 0.68c?

20.	With the help of a neat diagram, explain how the delta modulator tracks the analog input signal.

(OR)

21.	Explain how PRSQs are used for data encryption and data decryption.

22.	Explain in detail the operation sequence and buffering of RS 232

(OR)

23.	Explain a modem between DTE and phone line with it fundamental blocks

24.	Explain about cellular system implementation and brief about how does the cellular phone cross from cell to cell

(OR)

25.	What is CW Doppler? What radar information can it provide? What are it disadvantages?