Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	BASIC MECHANICAL ENGINEERING						Time : 3 hours

Subject Code:	ME106			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In 1872, James Starley, an Englishman developed a bicycle called __________.

2.	_______ was the inventor of the first automobile.

3.	Basically energy release is measured in terms of _______ and _______.

4.	The unit of energy in the SI system is _________.

5.	Power plants are broadly classified in to _____ and _____ power plants.

6.	The first nuclear reactor was built in the year _____ at Chicago USA.

7.	Split patterns will be assembled together in correct position by means of ______.

8.	Gas welding is also called as ______.

9.	In lathe, the ______ is used for thread cutting operation.

10.	________ is the best known rapid prototyping system

PART – B  (5 x 3 = 15 MARKS)

11.	List the purposes for which the ‘APPLE’ satellite was used.

12.	What are the important features of Babcock and Wilcox boiler?

13.	State the working principle of the hydro-electric power plant.

14.	What are the important properties required for good moulding sand?

15.	Name the various operations that can be performed on a lathe.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe the development history of ships.

(OR)

17.	Discuss the historical development of rockets.

18.	With a line sketch explain the various components of Cochran boiler.

(OR)

19. Explain with the aid of simple sketches, how a two stroke diesel engine works.

20.	Sketch and explain the various components of Thermal (Steam) Power Plant.

(OR)

21.	What are the advantages, disadvantages and applications of Solar Power Plant?

22.	Draw a neat sketch of cupola furnace and explain how cast iron is produced.

(OR)

23.	Explain with a sketch, the principle of gas welding process. Draw the three types of flames.

24. Sketch and explain the various parts of the standard shaping machine.

(OR)

25. Explain the principle parts of a centre lathe with a neat sketch.