Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MECHANICS OF MACHINES – II							Time : 3 hours

Subject Code:	ME208				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define a free body diagram

2.	What are dynamic forces?

3.	What is a turning moment diagram?

4.	The ratio of the maximum fluctuation of speed to the mean speed is called ………

5.	What is static balancing?

6.	The primary unbalanced force is maximum when the angle of inclination of the crank with the line of stroke is …………..

7.	Define frequency of a vibrating system

8.	The ratio of the maximum displacement of the forced vibration to the deflection due to the static force is known as …………

9.	Define torsional vibration

10.	At a nodal point is a shaft, the amplitude of torsional vibration is ……….

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes on piston effort

12.	Derive an expression for maximum fluctuation of energy in terms of mean kinetic energy of flywheel and coefficient of fluctuation of speed.

13.	What is variation of tractive force?

14.	Explain the terms, over damping, critical damping and under damping.

15.	Derive an expression for natural frequency of free torsional vibrations.

PART – C  (5 x 15 = 75 MARKS)

16.	A vertical double acting steam engine has a cylinder 300 mm diameters and 450mm stroke and runs at 200rpm. The reciprocating parts have a mass of 225kg and the piston rod is 50mm diameter. The connecting rod is 1.2m long. When the crank has turned through 125o from the top dead center, the steam pressure above the piston is 30 KN/m2  and below the piston is 1.5 KN/m2  calculate the effective turning moment on the crank shaft.

(OR)

17.	A connecting rod of an IC engine has a mass of 2 kg and the distance between the center of gudgeon pin and center of crank pin is 250mm. The C.G falls at a point 100mm from the gudgeon pin along the line of centers. The radius of gyration about an axis through the C.G perpendicular to the plane of rotation is 110mm. Find the equivalent dynamical system if only one of the masses is located at gudgeon pin.

If the connecting rod is replaced by two masses, one at the gudgeon pin and the other at the crank pin and the angular acceleration of the rod is 23000 rad/s2 clockwise, determine the correction applied to the system to reduce it to a dynamically equivalent system.

18.	A multi-cylinder engine is to run at a speed of 600rpm. On drawing the turning moment diagram to a scale of 1mm=250 N-m and 1mm = 30 , the areas above the mean torque line in mm2 are +160, -172, +168, -191, +197, -162.

The speed is to be kept with in ±1% of the mean speed of the engine. Calculate the necessary moment of inertia of the flywheel. Determine the suitable dimensions of a rectangular flywheel rim if the breath is twice its thickness. The density of the cast iron is 7250 Kg/m3 and its hoop stress is 6MPa. Assume that the rim contributes 92% of the flywheel effect.

(OR)

19.	A machine is coupled to a two-stroke engine which produces a torque of (800 + 180 sin 3) N.m where  is the crank angle. The mean engine speed is 400 rpm. The flywheel and the other rotating parts attached to the engine have a mass of 350kg at a radius of gyration of 200mm calculate.

a.	The power of the engine

b.	The total fluctuation of speed of the flywheel when

i)	The resisting torque is constant

ii).	The resisting torque is (800 + 80 sin) N.m

20.	Four masses A, B, C and D are completely balanced. Masses C and D makes angles of 90o and 1950 respectively with B is the same sense. The rotating masses have following particulars.

Mb = 25 Kg		ra = 150mm

Mc = 40 Kg		rb = 200mm

Md = 35 Kg		rc = 100mm

rd = 180mm

Planes B and C are 250mm apart. Determine

a.	The mass A and its angular position and

b.	The positions of planes A and D

(OR)

21.	A four cylinder vertical engine has cranks 150mm long the planes of rotation of the first, second and fourth cranks are 400mm, 200mm and 200mm respectively from the third crank and their reciprocating masses are 50Kg, 60Kg and 50Kg respectively. Find the mass of the reciprocating parts for the third cylinder and the relative angular position of the cranks in order that the engine may be in complete primary balance.

22.	In a single degree damped vibrating system, a suspended mass of 8 Kg makes 30 oscillations in 18 seconds. The amplitude decreases to 0.25 of the initial value after 5 oscillations. Determine

a.	The stiffness of the spring

b.	The logarithmic decrement

c.	The damping factor and

d.	`The damping coefficient

(OR)

23.	A shaft 40mm diameter and 2.5m long has a mass of 15kg per meter length. It simply supported at the ends and carries three masses 90kg, 140kg and 60kg at 0.8m, 1.5m, 2m respectively from the left support. Taking E=200 GN/m2 find the frequency of the transverse vibrations.

24.	The two rotors A and B are attached to the end of a shaft 985mm long. The mass of rotor A is 300kg with radius of gyration of 0.75m and the mass of B is 500kg with a radius of gyration of 0.9m. The shaft is 100mm diameter in first 300mm, 150mm in diameter for the next 160mm, 120mm diameter in next 125mm length and 90mm diameter in the last 400mm length. The modulus of rigidity for the shaft material is 84GN/m2 find

a.	The position of the node and

b.	The frequency of torsional vibration

(OR)

25.	A motor drives a centrifugal pump through gearing, the pump speed being one third that of the motor. The shaft from the motor to the pinion is 60mm diameter and 300mm long. The moment of inertia of the motor is 400kg-m2. The impeller shaft is 100mm diameter and 600mm long. The moment of inertia of the impeller is 1500kg.m2. Neglecting inertia of the gears and the shaft, determine the frequency of torsional vibration of the system, the modulus of rigidity of the shaft material is 80GN/m2.