Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MACHINE DESIGN & DRAWING							Time : 3 hours

Subject Code:	ME210			       						              		  Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Factor of safety should not be less than _________

2.	What is modulus of elasticity.

3.	Give any 2 material used for spring.

4.	Define spring.

5.	Shaft is a rotating member used to transmit ________

6.	What are the ‘2’ main type of couplings.

7.	The diameter of rivet hole & rivet will have __________ fit.

8.	What is welding.

9.	Flywheels stores____________.

10.	Crank shafts are used to convert rotory motion in to _________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define factor of safety. Compare this factor for brittle & ductile material.

12.	What are the advantages of helical spring.

13.	How the flexibility is obtained in flexible coupling.

14.	Define coefficient of sliding.

15.	What are the design parameters considered for piston.

PART – C  (5 x 15 = 75 MARKS)

16.	If a circular shaft is subjected to simple torsion, determine the design shear stress for the shaft from the data obtained from a simple tension test. The tension test gives elastic limit of material as 100N/mm2

(OR)

17.	What are the different mechanical properties of materials? Explain in detail.

18.	a.	Sketch a typical stress strain curve for ductile & brittle materials & explain.			(10)

b.	Write short notes on Fatique.													(5)

(OR)

19.	A helical spring made of C50 steel has an outside diameter of 80mm & a wire diameter of 12mm. The spring has to support a max. axial load of 1000N. Determine the max. shear stress & total deflection if the spring has 10.5 coils with ends ground flat. Determine also the factor of safety.

20.	Design a shaft to transmit power from an electric motor to a lathe head stock through a pulley by means of a belt drive. The pulley weighs 200 N & is located at 100mm from the centre of the bearing. Diameter of the pulley 200mm. Max power transmitted 1.5 H.P at 120 rpm. Angle of lap of belt 1800. Coefficient of friction between belt & pulley 0.3. Shock factor in bending 1.5. shock factor in twisting 2. Allowable shear stress in the shaft 35 N/mm2

(OR)

21.	Design a muff coupling to transmit 6 kW at 13rps. Design shear stress = 30 N/mm2 for shaft & key. Shear stress for sleeve is 15 N/mm. Crushing stress between shaft & key is 80 N/mm.

22.	An eccentrically loaded plate is welded to a frame as shown fig.

la

50

60 kN

80

lb

Design the welded joint if the tensile stress in the plate should not exceed 100 N/mm2 that in weld 80 N/mm2.

(OR)

23.	Design a knuckle joint to withstand a tensile load of 60 kN using steel with the following permissible stresses σt = 62.5 N/mm2 σc = 75 N/mm2, τ= 50 N/mm2.

24.	A double acting steam engine has the following specification.

Cylinder bore – 500 mm

Piston stroke  – 750 mm

Max. Pressure on Piston – 175 N/cm2

Connecting rod length – 1400 mm.

Select suitable material & design  the connecting rod.

(OR)

25.	Design a flywheel for punching machine which has a capacity of punching 30 holes of 20 mm diameter per minute in a steel plate 16 mm thick. The material of the plate has a ultimate shear stress of 350 N/mm2. The actual punching operation is estimated to last for a period of 36 degree of rotation of the punching machine crank shaft. This crank shaft is driven by flywheel shaft through a reduction gearing having ration 1;8. Assume mechanical efficiency of the punching machine is 80% permissible coefficient of fluctuation of speed is 0.15.