Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	GAS DYNAMICS AND JET PROPULSION					Time : 3 hours

Subject Code:	ME217			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the basic difference between compressible and incompressible fluid flow?

2.	Define stagnation state with example.

3.	Differentiate between nozzle and a diffuser.

4.	Sketch the shape of diffuser for supersonic flow, indicating the direction of fluid flow.

5.	Why Shock waves cannot develop in subsonic flow?

6.	Differentiate between normal and oblique shock.

7.	What is the significance of Mach number?

8.	Enumerate the conditions for fanno flow.

9.	Write the Euler’s equation for a reversible flow.

10.	How will you classify propulsive engines?

PART – B  (5 x 3 = 15 MARKS)

11.	Define Mach angle and Mach wedge.

12. 	Sketch the isentropic and adiabatic expansion process in P-V and T-S diagram.

13. 	Give the variation of Mach number across a convergent-divergent nozzle.

14. 	Give the sketch of Rankine-Hugoniot curve.

15. 	What is the difference between shaft propulsion and jet propulsion?

PART – C  (5 x 15 = 75 MARKS)

16.	Following quantities are given at the entry and exit of a passage:

Entry: P1=2.07 bar; T1=300 K; M1=1.4

Exit: M2= 2.5

Assuming isentropic flow of an ideal gas (γ=1.3; R=0.052 kJ/kg K). Determine

i.	Velocity of sound at the stagnation condition

ii.	The maximum velocity

ii.	The Mach number M*1 and M*2

iv.	Temperature and Pressure at exit.

(OR)

17.	An air jet has a velocity temperature equal to twice its static temperature. The static pressure and temperature are 2 bar and 300 K respectively. Determine

(i) 	Mach numbers M and M* of the air jet.

(ii) 	Stagnation temperature and Pressure

(iii)	Critical velocity of flow and sound.

[P.T.O]

18.	A conical diffuser has an inlet diameter of 40 cm and an exit diameter of 80 cm. Air enters the diffuser with a static pressure of 200 kPa, static  temperature 37°C and a velocity of 265 m/sec. Calculate

(i) Mass flow rate (ii) Properties at exit (iii) What will be the change in force exerted on the diffuser walls?

(OR)

19.	Air flowing at the rate of 1.15kg/sec with an approach velocity of 80 m/sec is expanded in a convergent-divergent nozzle. The static properties of air at inlet are 412 kPa and 160°C. The static pressure at exit is 103 kPa. Calculate the required throat and exit area for isentropic flow. If the isentropic efficiency of the nozzle is 85% and if the loss occurs only in the divergent section, find the loss in total head pressure.

20. 	Air enters an insulated tube of 0.03 m diameter through a convergent divergent nozzle with throat diameter 0.015 m. The stagnation properties of air at entry to nozzle are 750 kPa and

450 K. The flow through the nozzle is isentropic. The friction factor (4f) for the duct is 4f=0.02.Calculate the maximum length of tube and condition of air at exit if the length of pipe is 25 cm.

(OR)

21.	A gas (γ=1.3, R=0.287 kJ/ kg K, P1=1 bar and T1=400 K) enters a 30 cm diameter duct at a mach number of 2.0.A normal shock occurs at a mach number of 1.5 and the exit mach number is 1.0. If the mean value of the friction factor is 0.003,determine

(i) 	Length of the duct upstream and downstream of the shock

(ii)	Mass flow rate of the gas

(iii) 	Change of entropy upstream of the shock, across the shock and downstream of the shock.

22.	A convergent-divergent nozzle has an exit area to throat area ratio of 2. Air enters the nozzles with a stagnation pressure of 6.5 bar and a stagnation temperature of 93°C.The throat area is 6.25cm2. If there is a normal shock wave standing at a point where M=1.5, determine the pressure, temperature on either side of the plane. Find also the exit mach number of the nozzle.

(OR)

23. 	A Mach-2 aircraft engine employs a subsonic inlet diffuser of area ratio 3. A normal shock is formed just upstream of the diffuser inlet. The free stream conditions upstream of the diffuser are P0=0.10 bar and  T=300 K.

Determine:

i.	Mach number, pressure and temperature at the diffuser exit.

ii.	Diffuser efficiency including shock. Assume isentropic flow in the diffuser downstream of

the shock.

24. 	A turbo jet has a speed of 750 Km/hr while flying at an altitude of 10000 m. The propulsive efficiency of the jet is 50% and the overall efficiency of the turbine plant is 16%.The density of the air at 10000 m altitude is 0.173 kg/m3.The drag on the plane is 6250 N. Calorific value of the fuel is 48000 kJ/kg

Calculate

i.	Absolute velocity of the jet			ii.	Diameter of the jet

iii.	Power output of the unit in kW

(OR)

25. 	For a turbo jet with a flight velocity of 800 km/hr at an ambient of 60 kPa,the properties of gas entering the nozzle are 300 KPa and 200 °C. The mass flow rate of air is 20 kg/sec. Assuming air (γ=1.4; R=287 kJ/kg K) as working fluid, find

(i) Thrust developed				(ii) Thrust power			(iii)Propulsive efficiency.