Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	HEAT ENGINES AND FLUID MACHINERY		               Time : 3 hours

Subject Code: 	ME223			         						                Maximum Marks: 100

(Approved Heat and Mass Transfer Data Book is Permitted)

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define viscosity.

2.	What is vapour pressure?

3.	Turbine is a device which converts-----------------.

4.	The speed ratio of  Pelton wheel varies from --------------

5.	Define the term cycle.

6.	Why a heat engine cannot have 100% efficiency?

7.	Mention the various processes of  Brayton cycle.

8.	Differentiate between brake power and indicated power of an IC engine.

9.	What is meant by steady state heat conduction?

10.	The heat transfer in radiation takes place by means of -----------------

PART – B  (5 x 3 = 15 MARKS)

11.	Explain capillary action.

12.	What do you mean by governing of turbine?

13.	State the second law of thermodynamics

14.	Sketch the PV and TS diagram of diesel cycle and name all the process

15.	What is meant by absorptivity, reflectivity and transmissivity.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the construction and working principle of gear oil pump in detail with neat sketch.

(OR)

17.	A centrifugal pump having a outer diameter equal to two times the inner diameter and running at 1000 rpm works against a total head of 40 m. The velocity of flow through the impeller is constant and equal to 2.5 m/s. The vanes are set back at an angle of 40o at outlet. If the outer diameter of the impeller is 500 mm and the width at outlet is 50 mm, determine (i) vane angle at inlet (ii) work done by the impeller on water per sec and (iii) manometric efficiency.

18.	Explain the working principle of Kaplan turbine with neat diagram.

(OR)

19.	A Pelton wheel develops 8000 kW under a net head of 130 m at a speed of 200 rpm. Assuming the co-efficient of velocity for the nozzle 0.98, hydraulic efficiency 87%, speed ratio 0.46 and jet diameter to wheel diameter ratio 0.11, and the mechanical efficiency is 75%.  Determine

(i)The discharge required

(ii) the diameter of the wheel

(iii) the diameter and number of jets required, and

(iv) the specific speed

[P.T.O]

20.	In a steady flow apparatus, 135 kJ of work is done by each kg of fluid. The specific volume of the fluid, pressure and velocity at the inlet are 0.37 m3/kg, 600 kPa and 16 m/s. The inlet is 32 m above the floor and the discharge pipe is at floor level. The discharge conditions are 0.62 m3/kg, 100 kPa and 270 m/s. The total heat loss between the inlet and discharge is 9 kJ/kg of fluid. In flowing through this apparatus, does the specific internal energy increase or decrease, and by how much?

(OR)

21.	a.	Discuss the working principle of Carnot heat engine for stationary system with its

PV diagram. 																	(8)

b.	 A cyclic heat engine operates between a source temperature of 800oC and a sink temperature of 30oC. What is the least rate of heat rejection per kW net output of the engine? 		(7)

22.	In an air standard Otto cycle the pressure and temperature at the beginning of the cycle is 42oC and 0.1 MPa. The compression ratio and maximum temperature of the cycle are 8 and 1250oC respectively. Find (a) the temperature and pressure at the cardinal points of the cycle (b) heat supplied per kg of air (c) workdone per kg of air (d) cycle efficiency and (d) MEP of the cycle.

(OR)

23.	In a test of single cylinder diesel engine the following data are observed. Draw the energy balance sheet.

Load on the brake drums = 10 kg,

radius of the brake drum = 21 cm,

speed = 850 rpm,

fuel consumption = 0.670 kg/hr,

calorific value of the fuel = 44000 kJ/kg,

air consumption = 43.38 kg/hr,

cooling water rise in temperature = 15oC,

exhaust gas temperature = 325oC,

Cp of gas = 1.115 kJ/kg,

room temperature = 34oC and

mass flow rate of cooling water = 97.68 kg/hr.

24.	A wall is constructed of several layers. The first layer consists of masonry brick of 20 cm thick with thermal conductivity of 0.66 W/m K, the second layer consists of 3 cm thick mortar of thermal conductivity 0.6 W/m K, the third layer consists of 8 cm thick lime stone of thermal conductivity 0.58 W/m K and the outer layer consists of 1.2 cm thick plaster of thermal conductivity 0.6 W/m K. The heat transfer co-efficient of the interior and exterior of the wall is 5.6 W/m2K and 11 W/m2K respectively. Interior room temperature is 22oC and outside air temperature is - 5oC. Calculate

a.	Overall thermal resistance

b.	Overall heat transfer co-efficient

c.	Rate of heat transfer and

d.	Temperature at the junction between mortar and lime stone

(OR)

25.	Two parallel rectangular surfaces 1 m x 2 m are opposite to each other at a distance of 4 m. The surfaces are black and at 300oC and 200oC. Calculate the heat exchange by radiation between two surfaces.