Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ENGINEERING THERMODYNAMICS						Time : 3 hours

Subject Code:	ME228				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define System

2.	Volume is a -------- Property

3.	Cyclic integral of any property is equal to ----------.

4.	Heat Transfer is -----------function.

5.	Distinguish between ideal and real gas.

6.	Define calorific value.

7.	What is proximate analysis in combustion process?

8.	State the Avogadro’s law.

9.	Define compression ratio.

10.	Otto cycle is also called as -------------

PART – B  (5 x 3 = 15 MARKS)

11.	Determine the molecular volume of any perfect gas at 600 N/m² and 30°C.

Universal gas constant may be taken as 8314 J/kg-mole-K.

12.	Write the two statements of the second law of thermodynamics.

13.	Define critical temperature and temperature for water.

14.	Sketch the flow diagram of Rankine cycle indicating the main components.

15.	Define Stoichiometric air.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Prove that change in Entropy for polytrophic process is given by						(7)

ds = γ−nγ−1mR.lnv2v1

b.	Air in a closed vessel of fixed volume 0.15m³ exerts pressure of 12 bar at 250°C. If the vessel is cooled so that the pressure falls to 3.5 bar, determine

i.	Final temperature 															(2)

ii.	Heat transfer																	(3)

iii.	Change of entropy	   														(3)

(OR)

17.	Air flows steadily at the rate of 0.4 kg/s through an air compressor, entering at 6 m/s with a Pressure of 1bar and a specific volume of 0.85 m³/kg and leaving at 4.5 m/s with a pressure of 6.9 bar and a specific volume of 0.16m³/kg. The internal energy of the air leaving is 88kJ/kg greater than that of air entering. Cooling water in a jacket, surrounding the cylinder absorbs heat from the air at the rate of 59W. Calculate the power required to drive the compressor and inlet and outlet cross sectional areas

18.	a.	Prove that entropy is property.														(5)

b.	Establish the clausius in equality.                                                             				     (10)

(OR)

19.	Two reversible heat engines A and B are arranged in series, A rejecting heat directly to B.   Engine A receives 200KJ at a temperature of 421°C from a hot source, while the engine B is in communication with a cold sink at a temperature of 4.4 °C. If the work output of A is twice that of B. Find

a.	The intermediate temperature between A and B 										(5)

b.	The efficiency of each engine 														(5)

c.	The heat rejected to the cold sink 													(5)

20.	a.	Define the following terms:

i.	Quality of steam.  		ii.	Degree of Superheat. 		iii.	Degree of Sub cooling.

iv.	Triple Point  and Critical Point. 												(8)

b.	150 kg/Sec of steam at 25bar and 300°C expands isentropically in a steam turbine to        0.33 bar. Determine the power output of the turbine.               	 						(7)

(OR)

21.	1 kg of steam initially dry saturated at 1.1 MPa expands in a cylinder following the law PV1.13=C. The pressure at the end of expansion is 0.1 MPa. Determine

a.	The final volume																	(3)

b.	Final dryness fraction 																(3)

c.	Work done 																		(3)

d.	The change in internal energy 														(3)

e.	he heat transferred																(3)

22.	a.	State  Daltons Law of Partial Pressure.												(3)

b.	A Vessel of Volume 0.28m3 Contains 10 kg of air at 302 K. Determine the Pressure exerted by the air using.

i.	Perfect Gas Equation.

ii.	Vander Waals Equation.

iii.	Generalized Compressibility chart.

Take Critical Temperature of air is 132.8 K Critical Pressure of air is 37.7 bar.	           (12)

(OR)

23.	Explain the working principles of following calorimeter with neat sketch

a.	Bomb calorimeter	b.	Junkers calorimeter

24.	In a steam power plant the condition of steam at inlet to the steam turbine is 20bar and 300°C and the condenser pressure is 0.1bar. Two feed water heaters operate at optimum temperatures. Determine

a.	Quality of steam at turbine exhaust													(4)

b.	Net work per kg of steam															(3)

c.	Cycle efficiency 																	(4)

d.	Steam rate																		(4)

Neglect pump work

(OR)

25.	a.	Derive an expression for the air standard efficiency of Diesel cycle in terms of the

Compression ratio and Cut – off Ratio.		                        						     (10)

b.	In a Diesel cycle Compression ratio is 15 and fuel Cut- off is 5% of initial Volume. Find the

thermal efficiency.  				          											(5)