Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	THERMAL ENGINEERING						               Time : 3 hours

Subject Code: 	ME229			         						                Maximum Marks: 100

#### (Approved Steam Table, Mollier Diagram, Psychrometric Charts are Permitted)

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the use of flywheel?

2.	State any four functions of lubrication.

3.	Define nozzle efficiency.

4.	What are the different methods of compounding?

5.	What is clearance ratio?

6.	As the compression ratio increases the volumetric efficiency of air compressor will _________?

7.	Define tons of refrigeration.

8.	What are the properties of good refrigerant?

9.	Define wet bulb temperature.

10.	Define relative humidity.

PART – B  (5 x 3 = 15 MARKS)

11.	Draw the typical port timing diagram of a two stroke SI engine and explain its salient features.

12.	What are the effects of friction on the flow through a steam nozzle.

13.	How air compressors are classified?

14.	Draw the PV & TS diagram for vapour compression refrigeration cycle for super heating process.

15.	Name some psychrometric process and represent them on the psychrometric chart.

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Describe the valve timing diagram of four stroke CI engine with neat sketch.		(10)

b.	Compare two stroke and four stroke engine.										(5)

(OR)

17.	a.	Explain the working principle of magneto ignition system with a neat diagram.		(8)

b.	Describe the working principle of EGR system with a neat sketch.					(7)

18.	Stream approaches a nozzle with a velocity of 200 m/s, pressure of 4 bar and dryness fraction of 0.98. If the isentropic expansion in the nozzle proceeds till the pressure of exit is 1 bar. Determine the change in enthalpy and dryness fraction of steam using mollier diagram. Calculate also the exit velocity of steam from nozzle and exit area of nozzle for flow of 0.8 kg/s.

(OR)

19.	The velocity of steam leaving the nozzle of an impulse turbine is 1000 m/s and the nozzle angle is 20. The blade velocity is 350 m/s and the blade velocity coefficient is 0.85. Assuming no losses due to shock at inlet, calculate for a mass flow of 1.5 kg/s and symmetrical blading (a) blade inlet angle (b) driving force of the wheel (c) axial thrust on the wheel (d) power developed by the turbine.

[P.T.O]

20.	A single stage, single acting air compressor has a cylinder bore of 175mm and a stroke of 225mm. the compressor sucks air at 100 kN/m2 and 27C and delivers at 8 bar. Find the power required to drive a compressor if its speed is 120 rpm. Also calculate the mass of air compressed per minute and delivery temperature. The compression follows the law PV1.3 = C.

(OR)

21.	A two stage, single acting reciprocating air compressor with perfect intercooling with a suction pressure of 1 bar and temperature of 25C and final pressure is 5 bar. Compression and expansion follows the law PV1.3 = C. Find per kg of air (a) work done   (b) the mass of water necessary for abstracting the heat in the inter cooler, if the temperature rise of the cooling water is 30C.

22.	A vapour compression system works between the pressure limits of 58 bar and 25 bar. At the end of compression, the fluid is just dry, and there is not sub cooling. Determine the COP. Properties of the refrigerant is given below.

| Pressure bar | Saturation temperature, K | Enthalphy, kJ/kg |  | Entropy, kJ/kg |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Liquid | Vapour | Liquid | Vapour |
| 58 | 275 | 152 | 293.3 | 0.554 | 1.0332 |
| 25 | 261 | 56.32 | 322.6 | 0.226 | 1.2464 |

(OR)

23.	Explain the working principle of lithium bromide refrigeration system with neat sketch.

24.	An air conditioning system is designed under the following conditions.

Outdoor conditions	-	30C DBT, 75% RH

Required indoor conditions	-	22C DBT, 70% RH

Amount of free air circulated	-	3.33 m3s

Coil dew point temperature	-	14C

The required indoor condition is achieved first by cooling and dehumidification, and then by heating, Estimate (a) the capacity of cooling coil in tones and (b) the capacity of heating coil in kW.

(OR)

25.	Air at 20C, 40% relative humidity is mixed adiabatically with air at 40C, 40% relative humidity in the ratio of 1 kg of the former with 2 kg of the latter (on dry basis). Find the final condition of air.