Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	HEAT AND MASS TRANSFER						               Time : 3 hours

Subject Code: 	ME230			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Define thermal diffusivity of the material?

2.	What is meant by thermal resistance?

3.	What is the conduction shape factor?

4.	Lumped capacity approach can be used if Biot number ___________

5.	State Newton’s law of cooling?

6.	Define term effectiveness for a fin?

7.	When is the LMTD method most suitable for heat exchanger design calculations?

8.	What is the significance of Schmidt number?

9.	For a non reflecting surface ρ = _____ and  (τ+α ) = ____.

10.	What is pool boiling?

PART – B  (5 x 3 = 15 MARKS)

11.	Determine the loss of heat through the wall of a rotating sphere shaped boiling pan with an inner diameter d1= 0.5 m and the total boiler wall thickness of 20 cm . Inner surface temperature is    200 οC and that of the outer surface T2 is 50οC .The equivalent thermal conductivity is             0.12 W/m2K. Also find the heat flux?

12.	Brass wire of 0.5 mm diameter is annealed in a annealing furnace at a temperature of

500 οC. Find the time required for the wire to reach the temperature of 200 οC  if the temperature of the wire before passing into the furnace is 30 οC. Assume h= 30 W/m2K, = 600 kg/m3, specific heat as 0.4 kJ/kgK for brass and the  length of the wire is 10m?

13.	Explain the significance of non-dimensional numbers in convection heat transfer?

14.	What is Fouling? What are its types?

15.	Emissivities of two large parallel planes maintained at 800 οC  and 300 οC are 0.3 and 0.5 respectively. Find the net radiant heat exchange per square metre for these plates?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	A stainless steel wire of 3mm diameter, one metre long has a voltage of 100 volts impressed

on it. The outer surface of the wire is maintained at 100 οC. Calculate the centre temperature of the wire. The Resistivity of the wire is 10 μ Ohm –cm , thermal conductivity of the wire material is 20W/mC.the heated wire is submerged in a fluid maintained at 50 οC, find the heat transfer coefficient on the surface of the wire? 								(8)

b.	A 16 cm diameter pipe carrying saturated steam is covered by a layer of lagging of thickness 40 mm (K= 0.8 W/mC) . Later, an extra layer of lagging 10 mm thick (K= 1.2 W/mC)   is added. If the surrounding temperature remains the constant and heat transfer coefficient for both the lagging materials is 10 W/m2C, determine the percentage change in the rate of heat due to extra layers ?															(7)

(OR)

17.	a.	Estimate the rate of heat flow per m2 through a furnace wall consisting of 20cm thick inner

layer of chrome brick, a centre layer of kaolin brick 10 cm thick. The heat transfer coefficient at the inner surface is 74 W/m2 οC and the outer surface temperature is 70 οC.The temperature of the gases inside the furnace is 1670 οC. What temperatures prevail at the inner and outer surfaces of the centre layer? Assume  K(chrome brick)= 1.25 W/m οC,         K(kaolin brick)= 0.074 W/m οC and  K(masonary brick) = 0.555 W/m οC.			(10)

b.	A  small electric heating application uses a wire of 2mm diameter with 0.8mm thick insulation (K=0.12 W/mK).The heat transfer coefficient on the insulated surface(ho) is        35 W/m2K.Determine the critical thickness of insulation in this case  and the percentage of heat transfer rate if the critical thickness is used , assuming the temperature difference between the surface of the wire and the surrounding remain unchanged?			(5)

18.	a.	A large plate of aluminium 50 mm thick and initially at 200 οC  is suddenly exposed to the

convective environment at 70 οC. Calculate the temperature at a depth of 10 mm from one of the faces 1 minute after the plate has been exposed to the environment .Also find the energy removed from the plate per unit area during this period. Assume C= 0.9 kJ/kgK, K= 215 W/mK, h= 500W/m2C, α= 8.5 x 10-5 m2/s and ρ= 2700kg/m3.						(12)

b.  	A sphere of 4 m in diameter containing radioactive waste from nuclear reactor is buried in the earth. Its temperature at surface is 200 οC  and surface temperature of soil is 20 οC  (K soil = 0.3W/mC). Its center is 10 m below the surface of the soil. Determine the rate of heat loss from the sphere per hour if value of conduction shape factor is found out by the formulae (Fs= 4πr/(1-(r/2H)).													(3)

(OR)

19.	a.	One thousand spheres made of copper of diameter 5mm are annealed in the annealing

surface initial temperature of the sphere is 200C  and the temperature of the annealing furnace is 4000C.Assume h=30W/m2 0C,Cp=0.32 kJ/kgoC,ρ=3200 Kg/m3.Find the time required for the spheres to reach the temperature of 3000C.						(5)

b.	Explain in detail about the relaxation method used for solving 2 D heat

conduction problem. 															(10)

20.	a.	Air at 200C and 1atm pressure is flowing over a flat plate at a velocity of 3m/s.If plate is

30 cm wide and at 600C,Calculate the following quantities at x =30 cm:(i)Boundary layer thickness(hydrodynamic and thermal), (ii)Local and average friction coefficient (iii)Local and average heat transfer coefficient,(iv)Total heat loss/hour and total drag force on the plate.																		(10)

b.	Determine the flow of heat from a round iron rod of 10 cm length having diameter 1.6 cm. The base temperature of the rod is 300 0C and the atmospheric temperature is 300C.The thermal conductivity and the heat transfer coefficients are 40W/m 0C and 25W/m2 0C respectively.																	(5)

(OR)

21.	a.	A hot square plate 50 cm x 50 cm at 1000C is exposed to atmospheric air at 200C.Find the

heat loss from both  surfaces of the plate if (i)The plate is kept in vertical plane and (ii)A plate is kept in horizontal plane. Determine the percentage heat loss if the plate is kept horizontal instead of vertical.													(10)

b.	Consider an array of 8 fins used to cool a transistor. They are black anodized aluminium :

3 mm wide,0.4 mm thick and 5 cm long. When the base of the fins is at 60 οC  and the ambient is at 40 οC, how much power they dissipate? Assume h=10W/m2 0C and                 K= 180 W/mK.																	(5)

22.	a.	0.5 kg of ethylene glycol flows through a thin walled copper tube of 1.25 cm diameter

0.375 kg/s of water flows in opposite direction through the annular space formed by this tube and the tube diameter is 2 cm.The ethylene glycol which enters at  100 0C  is required to leave at 60 0C,while water enters at 10 0C .Calculate the length of the tube required if the water flows in the same direction  as ethylene glycol?	 							(12)

| S.No | FLUID PROPERTY | ETHYLENE GLYCOL AT 80oC | WATER AT 27oC |
| --- | --- | --- | --- |
| 1 | Density (kg/m3) | 1078 | 995 |
| 2 | Absolute viscosity(kg/ms) | 3200x 10-6 | 853 x10-6 |
| 3 | Specific heat (J/kgK) | 2050 | 4180 |
| 4 | Thermal conductivity(W/mK) | 0.261 | 0.614 |

b.	State the limitations of LMTD Method? 										(3)

(OR)

23.	a.	Co2 and air experience equimolar counter diffusion in circular tube whose length and

diameter are 1m and 50mm respectively. The system is at a total pressure of 1 atmosphere and a temperature of 250C.The ends of the tube are connected to large chambers in which the species concentration are maintain a fixed values. The partial pressure of Co2at one end is 190 mm of Hg while other end is 95mm of Hg. Estimate the mass transfer rate of Co2 and the air through the tube.															(7)

b.	The temperature of an air stream is to be measured, but the thermometer available does not

have a sufficiently high range. Accordingly, a damp cover is wrapped around thermometer before it is placed in the air stream .The thermometer readings is 220C. Estimate the true air temperature assuming it is dry at atmospheric pressure.							(8)

24.	a.	Two parallel plates of size 1m x 1m spaced 0.5m apart are located a very large room, the

walls of which are maintained at a temperature of 270C .One plate is maintained at 900 0C and other at 4000C .Their emmisivities are 0.2 and 0.5 respectively. If the plates exchanged heat between themselves and the surroundings, find the net heat transfer to each plate and to the room. Consider only the plate surfaces facing each other.						(10)

b.	State  Kirchoff  law and Stefan- boltzman law of radiation.						(5)

(OR)

25	a.	The surfaces of a double walled spherical vessels used for storing liquid oxygen are covered

with a layer of silver having an emmissivity of 0.03.The temperature of outer surface of the inner wall is –1530C and the temperature of the inner surface of the outer wall is 270C.The spheres are 21cm and 30cm respectively  in a diameter, with the space between them evacuated. Calculate the radiation heat transfer through the walls into the vessel and the rate of evaporation of the liquid oxygen if its rate of evaporation is 220 kJ/kg.			(5)

b.	Discuss briefly the various regimes in pool boiling heat transfer with a neat sketch.	(10)