Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	DESING OF MECHANICAL TRANSMISSION SYSTEMS      Time : 3 hours

Subject Code: 	ME231			         						                Maximum Marks: 100

(Assume necessary data’s & Use of approved Data Book and data sheets are permitted)

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Distinguish between long bearing and short bearing.

2.	Specify the purpose of crowning of flat pulley.

3.	What is the reason for using even number of pitches of length in chain?

4.	Define the module of gear.

5.	How are bevel gears classified?

6.	Write some applications of worm-gear drive.

7.	What is step ratio?

8.	What does the ray-diagram of gear-box indicate?

9.	Specify the types of threads used for power screws.

10.	Name few applications of Geneva mechanism.

PART – B  (5 x 3 = 15 MARKS)

11.	What are the desirable properties of bearing materials?

12. 	Distinguish between bush chain and roller chain.

13. 	What is meant by self-locking? At which gear it is available?

14.  	What are the possible arrangements to achieve 12 speeds from a gear box?

15. 	Write the formula which relates the weight of body, effort applied when the body is raised and lowered.

PART – C  (5 x 15 = 75 MARKS)

16.	The load on a 100 mm full hydro-dynamic journal bearing is 9000 N. speed of the journal is

320 rpm. Let the ratio of length of bearing to diameter of bearing is 1 and ratio of diametral clearance to diameter is 0.0011. The operating temperature = 65oC and minimum oil film thickness = 0.22 mm  (i) Select oil that will closely accord with the stated conditions. For the  selected oil, determine (ii) the friction loss (iii) the hydro dynamic oil flow through the bearing (iv) the amount of leakage (v) the temperature of oil passes through the bearing and (vi) maximum oil pressure.

(OR)

17.	Design a V-belt drive to drive a mill at 300 rpm and calculate the actual belt tensions and average stress for the following data. Pitch diameter of larger pulley is 500 mm, Pitch diameter of smaller pulley is 150 mm, Centre distance is 925 mm, motor speed is 1000 rpm and power to be transmitted is 7.5 kW.

18.	Design a pair of helical gears to transmit 10 kW at 1000 rpm of the pinion. Reduction ratio of 5 is required. Give details of the drive in a tabular form.

(OR)

[P.T.O]

19.	Design a chain drive to run a compressor from a 11 kW electric motor running at  970 rpm, the compressor speed being  330 rpm. The compressor operates 16 hours / day. The centre distance should be approximately 500 mm. The chain tension can be adjusted by shifting the motor on slides.

20.	The straight bevel gears are used in a speed reducer with a transmission ratio of 2. The wheel is supported on both sides and the pinion is over hanging. The input is from a 20 kW electric motor running at 950 rpm. Design the bevel gears.

(OR)

21.	The input to worm gear shaft is 18 kW and 600 rpm. Speed ratio is 20. The worm is to be of hardened steel and the wheel is made of chilled phosphor bronze. Considering wear and strength, design worm and worm wheel.

22.	The spindle of a pillar drill is to run at 12 different speeds in the range of 100 rpm and 355 rpm. Design a three stage gear box with a standard step ratio. Sketch the layout of the gear box, indicating the number of teeth on each gear. The gear box receives 5 kW from an electric motor running at 360 rpm. Sketch also the speed diagram.

(OR)

23.	A gear box is to be designed with the following specifications:

Power to be transmitted is 14.72 kW, number of speeds = 18, minimum speed 16 rpm,

Step ratio = 1.25, motor speed is 1400 rpm. The 18 speeds are obtained as 2 x 3 x 3. Sketch he layout of the gear box and the speed diagram. Calculate the diameter of the shafts and the number of teeth on the gears.

24.	Determine the geometric dimensions of a Geneva disc for the following data:

Distance between the centers is 60mm ,No of slots 6.Also calculate the power required to drive the wheel if angular velocity is 6.28 rad/sec and mass moment of inertia is 0.1 kg m2

(OR)

25.	The lead screw of a lathe has Acme threads of 50mm outside diameter and 10mm pitch. It drives the tools carriage and exerts an axial force of 2500N.A collar bearing with outside diameter 10cm and inside diameter 5cm is provided to take up the thrust. If the lead screw rotates at 30rpm find the efficiency and power required to drive the lead screw. Take coefficient of friction for screw threads as 0.15 and for collar as 0.12.