Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	STATISTICAL QUALITY CONTROL AND RELIABILITY IN MANUFACTURING

Time : 3 hours

Subject Code:	ME234			       			       						  Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In an ideal Six Sigma program, the maximum allowable Defects Per Million Opportunities (DPMO) is _____.

2.	Following numbers of defects were found while inspecting a sample of 10 mother-boards of a Laptop model in a company

1,2,4,2,2,3,5,1,4,2

What is the node of the above data?

3.	While selecting a subgroup for the purpose of constructing control charts, each subgroup should be as _________ (homogeneous/ heterogeneous) as possible.

4.	Name a type of control chart in which the control limits vary when the subgroup size varies from sample to sample.

5.	Refer the given Dodge-Romig table and choose the parameters of the single sampling plan for the following conditions:

Average Outgoing Quality Limit = 2%

Lot size =1000, Process average in percentage = 0.5.

6.	In a failure mode and effects analysis study, the severity, occurrence and detection were found to be 8, 7 and 9 respectively. Compute the Risk Priority Number.

7.	The history of failures experienced by the owner of a car is furnished below:

| Failure Number | Number of days from the date of purchase |
| --- | --- |
| 1 | 20 |
| 2 | 500 |
| 3 | 1200 |

Compute the Mean Time Between Failure of the car.

8.	What is the purpose for which “Quality Function Deployment’ technique is used?

9.	Draw a circuit diagram to depict element redundancy in a system?

10.	What can be the minimum and maximum values of maintainability?

PART – B  (5 x 3 = 15 MARKS)

11.	Write the Fiegenbaum’s definition of ‘Total Quality Control’.

12.	What are the information drawn from mean and range charts?

13.	The specification of a length of a die cast component is 100 ± 2mm.  The price of this component is Rs.100.  Determine Taguchi’s loss function.  From this function, determine the loss at 101mm.

14.	Name any three characteristics that are checked by conducting environmental tests.

15.	Consider a single component system which has a constant hazard rate of 0.8 units per hour and a mean down time of 1.5 hours.  What is the probability that the system is available at any time?

[P.T.O]

PART – C  (5 x 15 = 75 MARKS)

16.	a.	List any five activities assigned to a quality assurance function of a manufacturing 	organization.																	(5)

b.	Briefly explain the operation of a quality circle.									(10)

(OR)

17.	a.	A normal curve has an average of 140.6 and a standard deviation of 3.70. What percentage

of the area under the curve will fall between the limits of 135.5 and 142.5? 			(5)

b.	Using the Poisson and hyper-geometric attributes, compute the probabilities of zero through four defectives in a sample of 5 drawn from a lot of 40 which is 4% defective. 		(10)

18.	a.	Describe any two methods of calculating process capability. 						(5)

b.	In a capability study of a lathe used in turning a shaft to a diameter of 50.75±0.1mm a sample of 6 consecutive pieces was taken each day for 8 days.  The diameters of these shafts are as given below.  Construct X-bar and R chart and find out the process capability of the machine.  																	(10)

| 1st day | 2nd day | 3rd day | 4th day | 5th day | 6th day | 7th day | 8th day |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 50.77 | 50.8 | 50.77 | 50.79 | 50.75 | 50.78 | 50.76 | 50.76 |
| 50.8 | 50.78 | 50.78 | 50.76 | 50.78 | 50.76 | 50.78 | 50.75 |
| 50.78 | 50.76 | 50.77 | 50.79 | 50.78 | 50.73 | 50.75 | 50.77 |
| 50.73 | 50.7 | 50.77 | 50.74 | 50.77 | 50.76 | 50.76 | 50.72 |
| 50.76 | 50.81 | 50.80 | 50.82 | 50.76 | 50.74 | 50.81 | 50.78 |
| 50.75 | 50.77 | 50.74 | 50.76 | 50.79 | 50.78 | 50.8 | 50.78 |

(OR)

19.	a.	With the aid of an example, describe the concept of ‘chart sensitivity’.	  			(5)

- The following table gives the number of missing rivets noted at aircraft final

inspection.  																	(10)

| Air plane number | Number of missing rivets | Air plane number | Number of missing rivets | Air plane number | Number of missing rivets |
| --- | --- | --- | --- | --- | --- |
| 1 | 8 | 10 | 12 | 19 | 11 |
| 2 | 16 | 11 | 23 | 20 | 9 |
| 3 | 14 | 12 | 16 | 21 | 10 |
| 4 | 19 | 13 | 9 | 22 | 22 |
| 5 | 11 | 14 | 25 | 23 | 7 |
| 6 | 15 | 15 | 15 | 24 | 28 |
| 7 | 8 | 16 | 9 | 25 | 9 |
| 8 | 11 | 17 | 9 |  |  |
| 9 | 21 | 18 | 14 |  |  |

i.	Compute trial control limits of c chart for the above process.

ii.	Draw the c chart and plot the values.

iii.	What values of c-bar would you suggest for the subsequent period?

20.	Design a sequential sampling plan for the following specifications?

α = 0.05		p1 = 0.10

β = 0.20		p2 = 0.30

Also compute

i.	Average outgoing quality when p’ = p1

ii.	Minimum number of items inspected for accepting the lot.

iii.	Minimum number of defectives for rejection of the lot.

iv.	Average number of items inspected when the quality of the lot is P1

(OR)

21.	Consider a vacuum cleaner company with employee strength 1000. An overseas customer insists that the company has to implement ISO 9001:2000 based quality management system and obtain the certification from a reputed certifying agency.  List the steps that the company has to follow to carry out this task.

22.	a.	Following are the data gathered by conducting a reliability test on 10 picture

tubes of a TV.

| Month number | Cumulative failures | Number of survivors |
| --- | --- | --- |
| 0 | 0 | 10 |
| 1 | 2 | 8 |
| 2 | 5 | 5 |

Determine the failure rate of the picture tube during the first and second

months.  Calculate the mean failure rate of the picture tube for 2 months. 			 (5)

b.	Five elements (a,b,c,d and f) of a system are connected as shown in the following Figure which also indicates the reliabilities of each element. Calculate the system reliability.	(10)

f

b

c

d

a

0.8

0.7

0.7

0.9

0.8

(OR)

23.	a.	Describe the characteristics of Weibull model.										(5)

b.	Draw the framework of a FMEA table. ( Explanation / description is not necessary) 	     (10)

24.	a.	Consider a system in which two elements a1 and b1 are currently connected n series.  Now,

it is required to introduce unit redundancy by adding two elements a2 and b2 in the system.  Draw the circuits to indicate the two ways by which unit redundancy may be introduced.	(5)

b.	A system consists of three units connected in series with reliabilities R1 = 0.70, R2 = 0.80 and R3 = 0.90.  It is desired that the reliability of the system be 0.65.  How should this be apportioned among three units? 												     (10)

(OR)

25.	a.	Define maintainability.  Indicate the characteristics of maintainability.  					(5)

b.	With the aid of sketches, explain the concept of reliability and maintainability trade-off. (10)