Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	FLUID MECHANICS										Time : 3 hours

Subject Code:	ME251					         						 Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is Manometer

2.	What is the principle of Venturi meter

3.	What is continuity equation

4.	What is Stream function

5.	What is Characteristics curves of a pump

6.	What is water hammer

7.	What is Isentropic flow

8.	What is impulse function

9.	What is Oblique shock wave

10.	What is Jet propulsion

PART – B  (5 x 3 = 15 MARKS)

11.	Give different types of Pressure

12.	Write major and minor energy losses in pipe

13.	Why is NPSH

14.	What is Mach number

15.	What is Stagnation pressure ratio

PART – C  (5 x 15 = 75 MARKS)

16.	State Bernoulli’s theorem fro steady flow of an incompressible fluid. Derive an expression for

Bernoulli’s theorem from first principle and state the assumptions made for such a derivation.

(OR)

17.	a.	How would you determine the horizontal and vertical components of the resultant pressure 	on a sub-merged curved surface 													(8)

b.	Explain the procedure of finding Hydrostatic forces on curved surfaces 					(7)

18.	A fluid flow field is given by V=x2 yi + y2 zj – (2xyz + yz2)k.  prove that it is a case of possible steady incompressible fluid flow. Calculate the velocity and acceleration at the point ( 2, 1, 3)

(OR)

19.	A siphon of a diameter 200mm connects two reservoirs whose water surface level differs by 40m. The total length of the pipe is 8000m. The pipe crosses a ridge. The summit of ridge is 8 m above the level of water in the upper reservoir. Determine the minimum depth of the pipe below the summit of the ridge, if the absolute pressure head at the summit of siphon is not to fall below 3m of water. Take f = 0.006 and atmospheric pressure head = 10.3 m of water. The length of siphon from the upper reservoir to the summit is 500m. Find discharge also

20.	The internal and external diameter of an impeller of a centrifugal pump which is running at

1000rpm are 200mm and 400mm respectively. The discharge through pump is 0.04 m3/s and velocity of flow is constant and equal to 2m/s. the diameters of the suction and delivery pipes are 150mm and 100mm respectively and suction and delivery heads are 6m (abs) and 30m (abs) of water respectively. If the outlet vane angle is 450  and power required to drive the pump is 16.186 kW.Determine i) Vane angle of the impeller at inlet   ii) the overall efficiency of the pump  iii) Manometric efficiency of the pump

(OR

21.   a.	 With a neat diagram explain the working of a Reciprocating pump 			 (8)

b.	 Derive the expressions for Discharge through a reciprocating pump and work done by the reciprocating pump                           										 (7)

22.     Derive the expression for velocity of sound wave in fluid

(OR)

23.	a. 	Derive the expression for mass rate of flow of compressible fluid through

Venturi meter                                                                  				           	( 8)

b.	Find the mass rate of flow of air through a venture meter having inlet diameter 300mm and throat diameter 150mm. the pressure at the inlet of Venturimeter is 1.4kgf/cm2 absolute and temperature of air at the inlet 150C. The pressure at the throat is 1.3kgf/cm2 absolute. Take           R = 287J/kg.K and k= 1.4                                                  	                                      ( 7)

24.       Explain in detail flow with normal shock

(OR)

25.     Write a short note on	 a. Thrust equations 	 b. Liquid propellant

c. Turbo Jet components