Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MECHANICS OF MACHINES - I					               Time : 3 hours

Subject Code: 	ME254			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A  Rigid link is one which does not undergo any ____________while transmitting motion.

2.	Whitworth quick return motion mechanism is an inversion of____________________.

3.	For uniform Pressure the Mean radius(R) of friction surface in a clutch is ____________.

4.	A disc is spinning with an angular velocity ω rad/s about the axis of spin. The couple applied to the disc causing precession will be _____________

5.	The size of a cam depends upon ______________.

6.	For high speed engines, the cam follower should move with _________________motion.

7.	The maximum length of arc of contact for two mating gears, in order to avoid interference is _________

8.	When the axes of first and last gear are co-axial, then gear train is known as______________.

9.	A governor is said to be hunt if the speed of the engine fluctuates continuously____________mean speed

10.	The height of a watt’s governor (in meters) is equal to _________.

PART – B  (5 x 3 = 15 MARKS)

11. 	Differentiate between a Machine and Structure.

12.	Define axis of precession

13.	Write the classification of cams according to the surface contact.

14.	State law of gearing.

15. 	What is the difference between a governor and a flywheel?

PART – C  (5 x 15 = 75 MARKS)

16.	The crank and connecting rod of a steam Engine are 0.3m and 1.5m in length. The crank rotates at 180rpm clockwise. Determine the velocity and acceleration of the piston when the crank is at 400 from the inner deed centre position. Also determine the position of the crank for zero acceleration of the piston

(OR)

17.	The lengths of the crank and connecting rod of a horizontal reciprocating engine are 100mm and 500mm respectively. The crank is rotating at 400 rpm. Using Klein’s construction, find the angular acceleration of the connecting rod when the crank has turned 300 from the inner dead centre.

18.	A single dry plate clutch transmits 7.5 kW at 900 rpm. The axial pressure is limited to

0.07 N/mm2. If the coefficient of friction is 0.25, find i).Mean radius and face width of the friction lining assuming the ratio of the mean radius to the face width as 4  ii). Outer and inner radii of the clutch plate.

(OR)

[P.T.O]

19.	The turbine rotor of a ship has a mass of 2000 kg and rotates at a speed of 3000 rpm. Clockwise when looking from a stern. The radius of gyration of the rotor is 0.5 m.Determine the gyroscopic couple and its effects upon the ship when the ship is steering to the right in a curve of 100 m radius at a speed of 16.1 knots (1 knot = 1855 m/hr).

Calculate also the torque and its effects when the ship is pitching in simple harmonic motion, the bow falling with its maximum velocity. The period of pitching is 50 seconds and the total angular displacement between the two extreme positions of pitching is 120.Find the maximum acceleration during pitching motion.

20.	Construct the profile of a cam to suit the following specifications: Cam shaft diameter=40mm; Least radius of cam=25mm; Diameter of roller=25mm; Angle of lift=1200; Angle of fall =1500; Lift of the follower =40mm; Number of pauses are two of equal interval between motions. During the lift, the motion is S.H.M. During the fall, the motion is uniform acceleration and deceleration. The speed of the cam shaft is uniform. The line of stroke of the follower is off-set 12.5 mm from the centre of the cam.

(OR)

21.	A cam drives a flat reciprocating follower in the following manner: During first 1200 rotation of the cam, follower moves outwards through a distance of 20mm with simple harmonic motion. The follower dwells during next 300 of cam rotation. During next 1200 of cam rotation, the follower moves inwards with simple harmonic motion. The follower dwells for the next 900 of cam rotation. The minimum radius of the cam is 25mm. Draw the profile of the cam.

22.	Two involute gears of 200 pressure angle are in mesh. The number of teeth on pinion is 20 and the gear ratio is 2. if the pitch expressed in module is 5 mm and the pitch line speed is 1.2 m/s, assuming addendum as standard and equal to one module, find:

a. 	The angle turned through by pinion when one pair of teeth is in mesh; and

b. 	The maximum velocity of sliding.

(OR)

23.	In a reverted epicyclic gear train, the arm A carries two gears B and C and a compound gear

D-E. The gear B meshes with gear E and the gear C meshes with gear D. The number of teeth on gears B,C and D are 75,30 and 90 respectively. Find the speed and direction of gear C when gear B is fixed and the arm A makes 100 rpm.

24.	A Proell governor has equal arms of length 300 mm. The upper and lower ends of the arms are pivoted on the axis of the governor. The extension arms of the lower links are each 80 mm long and parallel to the axis when the radii of rotation of the balls are 150 mm and 200 mm. The mass of each ball is 10 kg and the mass of the central load is 100 kg. Determine the range of speed of the governor.

(OR)

25.	A loaded porter governor has four links each 250 mm long, two revolving masses each of 3 kg and a central dead weight of mass 20 kg. All the links are attached to respective sleeves at radial distances of 40 mm from the axis of rotation. The masses revolve at a radius of 150 mm at minimum speed and at a radius of 200 mm at maximum speed. Determine the range of speed.