Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MECHANICS OF MACHINES - II							Time : 3 hours

Subject Code:	ME255			       						                       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	A member under the action of three forces will be in equilibrium if the resultant of the forces is _________and the lines of action of the forces ________at a point.

2.	____________ is the net or effective force applied on the piston.

3.	The variations of energy above and below the mean torque line are called_________.

4.	The ratio of the maximum fluctuation of speed to the mean speed is ____________.

5.	A disturbing mass m1 attached to a rotating shaft may be balanced by a single mass m2 attached in the same plane of rotation as that of m1 such that__________.

6.	Partial balancing means______________.

7.	When the frequency of external exciting force is equal to the natural frequency of vibrating system, the amplitude of vibration is _________.

8.	Two springs of stiffness k1 and k2 are connected in series and mass m is attached to it. The natural frequency of longitudinal vibration will be ____________.

9.	The natural frequency (in Hz) of free torsional vibration is equal to __________.

10.	At a nodal point in shaft, the amplitude of torsional vibration is __________.

PART – B  (5 x 3 = 15 MARKS)

11.	Define inertia force and inertia torque.

12.	What is a flywheel? What is its use?

13.	What is meant by static and dynamic balancing?

14.	Distinguish between longitudinal, transverse and torsional vibrations.

15.	What is meant by torsionally equivalent shaft? State the expression for equivalent length of a 	shaft which has three steps.

PART – C  (5 x 15 = 75 MARKS)

16.	For the static equilibrium of the mechanism shown in Figure.1, find the required input torque at the crank. The dimensions of various links are:  OA = 250 mm, AB = 750 mm, and AC = 300 mm.

Figure .1	                                                        [P.T.O]

(OR)

17.	In a vertical double acting steam engine, the connecting rod is 4.5 times the crank. Weight of the reciprocating parts is 120 kg and the stroke of the piston is 440 mm. The engine runs at 250 rpm. The net load on the piston due to steam pressure is 25 kN when the crank is turned through an angle of 120° from the top dead centre, determine:

a.	the thrust in the connecting rod

b.	the pressure on side bars

c.	the tangential force on the crank pin

d.	the thrust on the bearings

e.	the turning moment on the crankshaft.

18.	The turning moment diagram of a four stroke engine is assumed to be represented by four triangles, the areas of which from the line of zero pressure are

Suction stroke 		=	 440 mm2

Compression stroke	=	1600 mm2

Expansion stroke		=	7200 mm2

Exhaust stroke		=	660 mm2

Each mm2 of area represents 3 N.m of energy. If the resisting torque is uniform determine the mass of the rim of a flywheel to keep the speed between 218 and 222 rpm when the mean radius of the rim is to be 1.25 m

(OR)

19.	The turning moment diagram for a multi cylinder has been drawn to a vertical scale of                1 cm = 5000 N-m torque and a horizontal scale of 1 cm = 60°. The intercepted areas between output torque curve and mean resistance line taken in order from one end are: - 0.3, + 4.1, - 2.8, + 3.2, -3.3, + 2.5, - 3.6, + 2.8 and - 2.6 cm2 when the engine is running at 800 rpm. The engine has a stroke of 30 cm and the fluctuation of speed is not to exceed 2% of the mean speed. Density of the material is 7.2 g/cm3 and width of the rim is 4 times its thickness. The centrifugal stress (hoop stress) in the rim material is limited to 280 x 105 N/m2. Neglecting the effect of boss and arms, determine the diameter and cross section of the flywheel rim.

20.	A shaft carries four masses A, B, C and D which are placed in parallel planes perpendicular to the longitudinal axis. The unbalanced masses at planes B and C are 3.6 kg and 2.5 kg respectively and both are assumed to be concentrated at a radius of 25 mm while the masses in planes A and D are both at radius of 40 mm. The angle between the planes B and C is 100° and between B and A is 190°, both angles being measured in counterclockwise direction from the plane B. The planes containing A and B are 250 mm apart and those containing B and C are    500 mm. If the shaft is to be completely balanced, determine:

- masses at planes A and D

- the distance between the planes C and D

- the angular position of the mass D

(OR)

21.The three cylinders of an air compressor have their axes 120° to one another and their connecting 	rods are coupled to a single crank. The stroke is 100 mm and the length of each connecting rod is 	150 mm. The mass of the reciprocating parts per cylinder is 1.5 kg. Find the maximum primary 	and secondary forces acting on the frame of the compressor when running at 3000 rpm. Describe 	clearly a method by which such forces may be balanced.

22.	a.	Determine the equation of motion for the system shown in Fig.2 					(5)

Figure. 2

b.	The mass of a single degree damped vibrating system is 7.5 kg and makes 24 free 	oscillations in 14 seconds when disturbed from its equilibrium position. The amplitude of 	the vibration reduces to 0.25 of its initial value after five oscillations. Determine:                  	a.	stiffness of the spring,  	b.	 Logarithmic decrement and 	c..	damping factor

(10)

(OR)

23.	Explain the following

a.	Vibration isolation and transmissibility										(8 )

b.	Whirling of shaft 															(7 )

24.	A steel shaft 1.5 m long is 95 mm in diameter for the first 0.6 m of its length, 60 mm in diameter for the next 0.5 m of the length and 50 mm in diameter for the remaining 0.4 m of its length. The shaft carries two flywheels at two ends, the first having a mass of 900 kg and 0.85 m radius of gyration located at the 95 mm diameter end and the second having a mass of 700 kg and 0.55 m radius of gyration located at the other end.  Determine the location of the node and the natural frequency of free torsional vibration of the system. Take modulus of rigidity of shaft material as 80GN/m2.

(OR)

25.	A reciprocating IC engine is coupled to a centrifugal pump through a pair of gears. The shaft from the flywheel of the engine to the gear wheel has 48 mm diameter and 800 mm long. The shaft from the pinion to the pump has 32 mm diameter and 280 mm long. Pump speed is four times the engine speed. Moments of inertia of flywheel, gear-wheel, pinion and pump impeller are 1000 kg-m2, 14 kg.m2 , 5 kg.m2 and 18 kg.m2 respectively. Find the natural frequency of the torsional oscillations of the system. Take modulus of rigidity as 80GN/m2.