Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title:	DESIGN OF MACHINE ELEMENTS						Time : 3 hours

Subject Code:	ME257				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is meant by Design?

2.	Bulk modulus is the ratio of ---------------- to ---------------------.

3.	What are the factors affecting endurance limit?

4.	Give an example of a machine member subjected to bending and torsional stresses.

5.	When the material is loaded within elastic limit, then the stress is -------------------to strain.

6.	Two shafts will have equal strength if --------------------------- for both are same.

7.	A key made from a cylindrical disc having segmental cross-section is known as--------------------.

8.	What is the function of a coupling?

9.	State some applications of cotter joints?

10.	Of what materials is I.C. engine pistons generally made?

PART – B  (5 x 3 = 15 MARKS)

11.	What are the factors to be considered in the selection of materials for machine members?

12.	Given four examples of applications for the following springs

a.	Helical springs 	b.	Leaf springs		c.	Torsional springs 	d.	Disc springs.

13.	Distinguish between shaft, axle, and spindle from the design point of view.

14.	What kind of failures will occur in a riveted joint and how can they be rectified?

15.	What is slenderness ratio? Explain its significance in the design of connecting rod?

PART – C  (5 x 15 = 75 MARKS)

16.	The crane hook carries a load of 20kN, as shown in Fig 1. The section at X-X is rectangular whose horizontal side is 100mm. Find the stresses in the inner and outer fibers at the given section.

Fig 1

(OR)

17.	An overhang crank with pin and shaft is shown in Fig 2. A tangential load of 15kN acts on the crane pin. Determine the maximum principal stress and the maximum shear stress at the centre of the crankshaft bearing.

Fig 2

18.	A hot rolled steel shaft is subjected to a torsional moment that varies from 330 N-m clockwise to 110 N-m counterclockwise and an applied bending moment at a critical section varies from     440 N-m to -220 N-m. The shaft is of uniform cross-section and no keyway is present at the critical section. Determine the required shaft diameter. The material has an ultimate strength of            550 X106 N/m2  and a yield strength of 410 X106 N/m2. Take the endurance limit as half the ultimate strength, factor of safety of 2, size factor of 0.85 and a surface finish factor of 0.62.

(OR)

19.	Design a helical compression spring for a maximum load of 1000 N for a deflection of 25 mm using the value of spring index as 5.

The maximum permissible shear stress for spring wire is 420 N/mm2 and modulus of rigidity is 84 KN/ mm2.

Take Wahl’s factor,   K =  4C−14C−4+0.615C, where C = spring index.

20.	Design a shaft to transmit power from an electric motor to a lathe head stock through a pulley by means of a belt drive. The pulley weighs 200 N and is located at 300mm from the centre of the bearing. The diameter of the pulley is 200 mm and the maximum power transmitted is 1 KW at 120 r.p.m. The angle of lap of the belt is 180° and coefficient of friction between the belt and the pulley is 0.3. The shock and fatigue factors for bending and twisting are 1.5 and 2.0 respectively. The allowable shear stress in the shaft may be taken as 35 N/ mm2.

(OR)

21.	Design and draw a protective type of cast-iron flange coupling for a steel shaft transmitting 15 kW at 200 r.p.m. and having an allowable shear stress of 40 N/ mm2. The working stress in the bolts should not exceed 30 N/ mm2. Assume that the same material is used for shaft and key and that the crushing stress is twice the value of its shear stress. The maximum torque is 25% greater than the full load torque. The shear stress for cast-iron is 40N/ mm2.

22.	A double riveted lap joint with Zig-zag riveting is to be designed for 13 mm thick plates. Assume ft = 80 N/ mm2, fs =60 N/ mm2 , and  fc =60 N/ mm2.. State how the joint will fail and find the efficiency of the joint.

(OR)

23.	Design a knuckle joint for a tie rod of a circular section to sustain a maximum pull of 70 kN. The ultimate strength of the material of the rod against tearing is 420 N/ mm2. The ultimate tensile and shearing strength of the pin material are 510 N/ mm2 and 396 N/ mm2 respectively. Determine the tie rod section and pin section. Take factor of safety = 6.

24.	Calculate the diameter of a piston rod for a cylinder of 1.5 m diameter in which the greatest difference of steam pressure on the two sides of the piston may be assumed to be 0.2 N/ mm2. The rod is made of mild steel and is secured to the piston by a tapered rod and nut and to the crosshead by a cotter. Assume modulus of elasticity as 200 kN/ mm2 and factor of safety as 8. Length of the rod may be assumed as 3 metres.

(OR)

25.	The turning moment diagram for a petrol engine is drawn to the following scales:

Turning moment, 1 mm = 5 N-m Crank angle, and 1 mm = 1°.

The turning moment diagram repeats itself at every half revolution of the engine and the areas above and below the mean turning moment line, in order are 295, 685, 40, 340, 960, 270 mm2.

Determine the mass of 300 mm diameter flywheel rim when the coefficient of fluctuation of  the width of the rim is twice of thickness. Assume density of rim material as 7250 Kg/m3.