Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MANUFACTURING TECHNOLOGY				               Time : 3 hours

Subject Code: 	ME258				   						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	___________serve as reservoirs to supply the molten metal necessary to prevent shrinkage during solidification.

2.	What is the function of a core?

3.	____________is a process in which the work piece is shaped by compressive forces applied through various dies and tools.

4.	Mention any two applications of stretch forming.

5.	__________in welds is caused by gases released during melting of the weld area but trapped during solidification.

6.	What is fusion welding?

7.	A _________chip is obtained when cutting ductile materials.

8.	Define cutting ratio.

9.	__________is the operation of cutting internal threads by means of cutting tool called tap.

10.	Define broaching operation.

PART – B  (5 x 3 = 15 MARKS)

11.	Name the important factors in selecting pattern materials.

12.	What are the various extrusion defects?

13.	What is meant by solid state welding?

14.	State the functions of a cutting fluid.

15.	What is Honing and state its functions.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the investment casting process with sketches.

(OR)

17.	a.	What are the common defects in casting?										(5)

b.	Explain the shell-mold casting process with sketches.								(10)

18.	Explain the principles of different forging machines.

(OR)

19.	Explain the various shearing operations on sheet metal with neat sketches.

20.	Explain the various techniques used for testing welded joints.

(OR)

21.	Describe the principle of friction welding with a neat sketch.

22.	Discuss the various forces encountered in metal cutting.

(OR)

23.	How are automatic lathes classified? Briefly explain any one of those machines.

24.	Sketch and describe a horizontal milling machine.

(OR)

25.	a.	What are the factors to be considered in selection of a grinding wheel?				(5)

b.	Sketch and explain the working of an external cylindrical grinding machine.			(10)