Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ADVANCED MANUFACTURING TECHNOLOGY			Time : 3 hours

Subject Code:	ME259				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Name any two methods of powder production in powder metallurgy.

2.	The first step in processing ceramics is ____________

3.	Blow moulding is a modified ___________ and ___________ moulding process.

4.	Reinforced plastics consist of ___________ in a ___________ matrix.

5.	Electro-chemical machining is basically the reverse of ____________

6.	In abrasive jet machining, a jet of ______ or _______ containing abrasive particles is aimed at the work piece surface.

7.	Single crystal silicon is almost always obtained by using the _________ process.

8.	What do CVD stand for in film deposition?

9.	Symbols like G01,G02 etc used in programming are called _________________

10.	In CNC, the commonly used interpolation methods are _________ and ________

PART – B  (5 x 3 = 15 MARKS)

1.	What is the difference between ‘Impregnation’ and ‘Infiltration’?

2.	What types of polymers are most commonly blow moulded? Mention any three types.

3.	List the functions of dielectric fluid in EDM.

4.	What are the purposes for ‘pre baking’ and ‘post baking’ in lithography?

5.	Distinguish between absolute and incremental programming. Give an example for each.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe briefly the production steps involved in making powder-metallurgy

parts.

(OR)

17.	Explain the various operations involved in processing ceramics.

18.	Describe the process of injection moulding with suitable sketches and state its

advantages.

(OR)

19. Discuss the major design considerations in forming and shaping plastics.

20.	What is electro-chemical machining? What are its applications, advantages and

disadvantages?

(OR)

21.	Describe the working principle of Electrical Discharge Machining (EDM) with a neat diagram? What are its applications?

22.	Write short note on the following terms: a) Single crystal growing b) Wafer slicing and c) Film Deposition.

[P.T.O]

(OR)

23.	Write short note on the following terms:

a.	Lithography 		b.	 Etching and 		c.	 Ion-Implantation and diffusion.

24.	Discuss the following CNC control systems with an example:

a.	point-to-point	 b.	 straight line and	c.	 contouring/continuous path.

(OR)

25.	Briefly explain with a simple sketch, the basic robot elements.