Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	DESIGN OF HEAT EXCHANGERS AND PRESSURE VESSELS FOR FOOD PROCESSING

Time : 3 hours

Subject Code:	ME261		       						                              Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Draw the temperature distribution in a evaporator?

2.	Write the LMTD formulae for counter flow double pipe heat exchanger?

3.	What is the need for correction factor in Multi pass shell & tube Heat exchanger/?

4.	Why baffles are used in shell and tube heat exchangers?

5.	State any one advantage of induced draught fan over forced draught fans?

6.	Name at least two materials used for cooling towers?

7.	______________ air preheaters are also known as storage type preheaters..

8.	What is working principle of recuperative air preheaters?

9.	What is meant by fouling?

10.	An evaporator is also known as ____________coil.

PART – B  (5 x 3 = 15 MARKS)

11.	Define overall heat transfer coefficient ? What is its significance ?

12.	Find the surface area required for a surface condenser dealing with 30000kg of steam per hour, 90% dry with latent heat of vaporization = 2440kJ/kg .The cooling water enters the condenser at 150C and leaves at 250C.Assume overall heat transfer coefficient as 3000 W/m2K.

13.	State the advantages of air cooled condenser over water cooled condenser?

14.	Explain the process of batch cooling of water with external heat exchanger using isothermal cooling medium?

15.	Define the capacity of evaporator? What are the modes of heat transfer possible in an evaporator?

PART – C  (5 x 15 = 75 MARKS)

16.	a.	Find the length of the tube required for the following heat transfer where air is heated by

exhaust gases. The inside and outside diameters of the tube are 5 and 6 cm respectively with heat transfer rate of 8000W.The inside and outside heat transfer coefficient on air and gas side are 100 and 160 W/m2K respectively. Temperature are:Thi=35 0C,  Tho=150 0C, Tci=50 0C and Tco= 100 0C.Neglect the tube resistance and assume flow arrangement is parallel. If the flow is made counter then what is the percentage saving in tube length?

(4+3)

b.	A heat exchanger has 17.5 m2 area available for heat transfer .It is used for cooling coil at 2000C by using water available at 20 0C.The mass flow and specific heat of oil are 10000kg/hr and 1.9 kJ/kgK and the mass flow and specific heat of water are 3000kg/hr and 4.18 kJ/kgK respectively. If the overall heat transfer coefficient is 300 W/m2k, estimate the outlet temperature of oil and water for parallel flow and counter flow arrangement by LMTD method?																		(8)

(OR)

17.	0.5 kg of ethylene glycol flows through a thin walled copper tube of 1.25 cm diameter 0.375 kg/s of water flows in opposite direction through the annular space formed by this tube and the tube diameter is 2 cm. The ethylene glycol which enters at  100 0C  is required to leave at 60 0C,while water enters at 10 0C .Calculate the length of the tube required if the water flows in the same direction  as ethylene glycol? Properties of fluids are as below:

| S.No | FLUID PROPERTY | ETHYLENE GLYCOL AT 80oC | WATER AT 27oC |
| --- | --- | --- | --- |
| 1 | Density (kg/m3) | 1078 | 995 |
| 2 | Absolute viscosity(kg/ms) | 3200x 10-6 | 853 x10-6 |
| 3 | Specific heat (J/kgK) | 2050 | 4180 |
| 4 | Thermal conductivity(W/mK) | 0.261 | .614 |

18.	a.	Fuel oil at the rate of 1.1kg/s is heated passing through the annulus of a counter flow double

pipe heat exchanger from 100C to 200C by using hot water available from the engine at 710C.Water flows through a copper tube (OD =2.13 cm and ID=1.86 cm) with a velocity of 0.76m/s the oil passes to the annulus formed by inner copper tube and outer steel pipe(OD=3.34 cm & ID=3 cm).The fouling factor is 0.0004 m2 C/W and  0.0009 m2 C/W on water and oil side respectively. Neglect the resistance of the copper tube.			   (12)

| S.No | FLUID PROPERTY | OIL | WATER |
| --- | --- | --- | --- |
| 1 | Density (kg/m3) | 854 | 982 |
| 2 | Kinematic viscosity(m2/s) | 7.43x10-6 | 4.18x10-7 |
| 3 | Specific heat (J/kgK) | 1884 | 4187 |
| 4 | Thermal conductivity(W/mK) | 0.138 | 0.657 |

b.	Discuss the use of baffle and its types?												(3)

(OR)

19.	a.	Draw the different configurations of shell and tube heat exchanger.                                    (5)

b.	Draw the standard pattern of tube layout used in shell and tube heat exchanger.                 (5)

c.	Why we need correction factor in the design of shell and tube heat exchanger? Explain.    (5)

20.	a.	Discuss the design features of condenser in detail.                                                              (10)

b.	Explain in detail the pressure drop and Pumping power calculation in double pipe heat

exchanger.                                                                                                                              (5)

(OR)

21.	a.	Explain the working of mechanical draught cooling towers? 						       (7)

b.	Compare the forced draft tower with induced draft tower?								(8)

22.	Discuss in detail recuperative and regenerative air preheaters?

(OR)

23.	a.	Explain the heating of water with external heat exchanger using isothermal heating

Medium?																		(6)

b.	Find the time to heat a liquid from initial temperature Tci to final temperature Tco in a tank

which is uniformly agitated with help of steam?										(9)

24.	a.	Classify the evaporators.															(5)

b.	Explain in detail the working of multi-effect evaporators? 							     (10)

(OR)

25.	a.	What are the factors affecting the heat transfer capacity of an evaporator?				(5)

b.	Explain the working of single effect evaporators.									     (10)