Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MECHANICAL SYSTEMS FOR FOOD PROCESSING		Time : 3 hours

Subject Code:	ME262		         						                	 Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1. 	What is slip in relation to a reciprocating pump?

2.	What is the function of a volute in a centrifugal pump?

3. 	Name the causes for water hammering in a pump.

4. 	What is a timing belt? Where is it used?

5.	What is dryness fraction?

6. 	What is an IBR and non IBR Boiler.

7. 	What is latent heat?

8. 	What is brine?

9. 	Lancashire boiler is of ---------------type based on furnace position

10. 	Name the methods to activate the non-powered conveyors

PART – B  (5 x 3 = 15 MARKS)

Part –B ( 5X3=15 Marks)

11. 	What is cavitaion in pumps? How can it be prevented?

12. 	Give any four advantages and disadvantages of using V-belts.

13.	Differentiate between the economizer and super heater

14.	What are the types of chilled water generators?

15. 	Enumerate the advantages and disadvantages of pneumatic conveyors.

PART – C  (5 x 15 = 75 MARKS)

16.	 A centrifugal pump delivers water against a net head of 14.5 meters and a design speed of 1000 rpm. The vanes are curved back to an angle of 30 0 with the periphery. The impeller diameter is 300 mm and outlet width 50 mm. Determine the discharge of the pump if manometric efficiency is 95%.

(OR)

17.	a.	Explain the working of a gear pump with a neat sketch.  						(7)

b.	Explain the working of a peristaltic pump. Give its advantages.				(8)

18.	a.	A solid shaft is required to transmit 120 kW power at 200 rpm. Find the suitable diameter

of the shaft if the maximum torque transmitted in each revolution exceeds the mean by 20%.Take allowable shear stress as 70 N/mm2 for the material of the shaft.		(10)

b.	Name the types of couplings used in major turbo machinery. Mention any two advantages of gear type coupling.   														(5)

(OR)

19.	 A belt is running over a pulley of diameter 120 cm at 200 rpm. The angle of contact is 165 o

and co-efficient of friction between the belt and pulley is 0.3. If the maximum tension in the

belt is 3000 N , find the power transmitted  by the belt.

20.	a.	Explain the working of a spring loaded safety valve. 							(6)

b.	With a neat sketch explain the working of a Babcock and Wilcox boiler. 		(9)

(OR)

21.	a.	Explain with neat sketches the functioning of Benson Boiler. Give its merits and

demerits.(10)

b.	Compare the fire tube boiler and water tube boiler.        						(5)

22.	 a.	.Explain with sketches any   three types of water coolers 						(8)

b.	 Briefly explain three common types of ice storage systems					(7)

(OR)

23. 	A food storage chamber requires a refrigerant system of 10 tonnes capacity with an evaporator

temperature of -8 0 C  and condenser  temperature of 30 o C. The refrigerant  F – 12 is sub cooled

by 5o C before  entering  the throttle  valve , and vapour  is superheated  by  6o C  before  entering the compressor. The specific heats of vapour and liquid are 0.7327 and 1.235 respectively. Determine a) the refrigerating effect per kg, b) mass of refrigerant circulated per minute , and c)  C.O.P. The properties of the refrigerant are :

| Temperature in oC | Enthalpy kJ/kg |  | Entropy in kJ/kg K |  |
| --- | --- | --- | --- | --- |
|  | Liquid h | Vapour H | Liquid φl | Vapour φv |
| -8 | 411.289 | 569.808 | 4.1618 | 4.7564 |
| 30 | 447.883 | 586.515 | 4.2874 | 4.7438 |

24.	a.	Explain the working of a bucket elevator system with a neat sketch.			(8)

b.	Briefly explain the working of an overhead trolley conveyor and mention few of its advantages. 																(7)

(OR)

25. 	Expalin the functioning of (i) bridge, (ii) gantry and (iii) jib cranes. 			 (5+5+5)