Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	FINITE ELEMENT ANALYSIS						               Time : 3 hours

Subject Code: 	ME269			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The weighting function for the subdomain method is ___________.

2.	Define functional.

3.	Define degrees of freedom.

4.	Calculate the half bandwidth for the one-dimensional finite element model shown in Fig. 1.

1

4

2

3

Fig. 1

5.	The value of shape function equals to __________ at its own node.

6.	What is C0 continuity?

7.	Lagrangean elements are more accurate than Serendipity elements. (True/False)

8.	Multiplex element is also a complex element. (True/False)

9.	State the element equation for one-dimensional heat conduction.

10.	Mention at least four commercial FEA packages.

PART – B  (5 x 3 = 15 MARKS)

11.	Explain the basis for weighted residual methods.

12.	Draw the Pascal’s triangle and select a suitable interpolation equation for an 8-noded quadrilateral element.

13.	What is the significance of element connectivity table?

14.	Explain the term isoparametric formulation.

15.	Distinguish between plane stress and plane stain analysis.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the step – by – step procedure of finite element method with a suitable example.

(OR)

17.	Consider a 1mm diameter, 50 mm long aluminium pin-fin used to enhance the heat transfer from a surface wall maintained at 300º C. The governing differential equation

and boundary conditions are given by

kd2Tdx2=PhAC(T−T∞)T(0)=TW=300°CdTdx(L)=0(insulated tip)

[P.T.O]

where, k = thermal conductivity, P = perimeter, Ac = cross-sectional area, h = convective heat transfer coefficient, Tw = wall temperature, T∞ = ambient temperature. Let k = 200 W/m K for aluminium, h= 20 W/m2 K, T∞= 30 º C. Estimate the temperature distribution in the fin using the Galerkin weighted residual method.

2 (50, 40)

q’1

q’2

x

y

Fig. 2

18.	Consider the truss element shown in Fig. 2. The x-y coordinates of the two nodes are indicated in the figure. If q = [0.15, 0.1, 0.21, 0.2]T mm, determine the stress and strain energy in the element. Take young’s modulus, E = 200 GPa and cross-sectional area, Ac = 1250 mm2.

(OR)

19.

5 kN

3 m

2 m

Fig. 3

Calculate the global stiffness matrix of the cantilever beam shown in Fig. 3. Assume                     E = 2 x 105 MPa and I = 4 x 106 mm4.

20.	How is the order of interpolation equations selected? Also, explain the convergence and continuity requirement for displacement based finite elements.

(OR)

21.	For the triangular element shown in Fig. 4, the nodal values of displacement are:

u1 = 2.0	u2 = 3.0	u3 = 5.0

v1 = 1.0	v2 = 2.0	v3 = 3.0

Obtain the displacements of point P(2, 2) within the element.

(1,1)  1

2  (3,1)

3 (2, 3)



v

u

pu

Fig. 4

22.	Derive the shape function equations for an eight node quadrilateral serendipity element in natural coordinate (ξ, η) system.

(OR)

23.	Write short notes on the following:

a.	Numerical integration.	b.	Lagrange and serendipity elements

24.	Determine the temperature distribution in the rectangular fin. The length of the fin is 0.05 m and the cross-section is 0.02 m × 0.01 m. The wall temperature is maintained at 140º C. The thermal conductivity of the fin material is 7000 W/m K. Assume steady state, and only conduction process. Take heat generated inside the fin as 4000 W/m3.

(OR)

25.	Derive the [B] matrix for a CST element. Also calculate the strain-displacement matrix for the element shown in Fig.5.

1 (3, 5)

2 (6, 4)

3 (4.5, 7)

Fig. 5