Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MECHANICAL VIBRATIONS								Time : 3 hours

Subject Code:	ME270			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is the difference between a discrete and continuous system?

2.	If a system vibrates due to an external excitation, it is called as __________vibration.

3.	What is damped Vibration?

4.	What is logarithmic decrement?

5.	What are flexibility influence coefficients?

6.	What is modal analysis?

7.	Why a continuous system is called as a system of infinite degree of freedom?

8.	The mode corresponding to n = 1 is called as _______mode.

9.	Most Mechanical frequency measuring instruments are based on the principle of ______.

10.	The life of a machine follows the classic __________curve.

PART – B  (5 x 3 = 15 MARKS)

11.	Motion of a simple pendulum is an example of single degree freedom system. Justify.

12.	State some characteristics of coulomb damping.

13.	What are the factors to be considered for choosing the type of vibration measuring instruments in a vibration test.

14.	State some methods to control vibrations.

15.	Briefly explain a band pass filter.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain the construction of viscous dampers with a neat figure.

(OR)

17.	Explain the various procedures involved in the analysis of a vibration system.

18.	Derive D’ Alembert’s dynamic equilibrium equatin.

(OR)

19.	Use principle of conservation of energy to derive the equation of motion.

[P.T.O]

20.	Estimate the fundamental frequency of vibration of the system shown in figure (1)

k1

m1

x1(t)

m2

m3

x2(t)

x3(t)

k2

k3

Assume that m1 = m2 = m3 = m, k1 = k2 = k3 = k and the mode shape is x=12

(OR)

21.	Derive the orthogonality principle for normal functions for the longitudinal vibration of bars.

22.	Derive the equation of motion and solution for the longitudinal vibration of a rod of length 1 with varying cross section.

(OR)

23.	Determine the natural frequencies of vibration of a uniform bar from clamped at one end and free at the other.

24.	With a schematic diagram explain the principle and working of an electro dynamic shaker. Represent the typical resonance characteristics.

(OR)

25.	Explain with a diagram how a strain gauge could be used as an vibration pick up.