Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	TRIBOLOGY IN DESIGN									Time : 3 hours

Subject Code:	ME272				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What are the different features that surface texture is composed of?

2.   What happens if the rate of change of the coefficient of kinetic friction as a function of sliding

velocity is negative?

3.	Specify the conditions under which stress corrosion cracking occurs

4.	Define erosion ratio

5.	What factors govern the friction and wear under boundary lubricated conditions?

6.	Give examples of solid lubricants

7.	Suggest a suitable bearing for an application that calls for (a) less sensitiveness to interruption in lubrication (b) less sensitiveness to misalignments

8.	What is the cause of synchronous whirl in a hydrodynamic journal bearing?

9.	What are the desirable tribological properties of materials used for rolling element bearings?

10.	Give any two typical compositions of babbit alloys

PART – B  (5 x 3 = 15 MARKS)

11.	The roughness heights of a surface, measured from a mean centerline are as follows: 15, 20, 33, 18, 29 microns. Determine the rms value of surface roughness.

12.	Give four examples of components which are vulnerable to fretting wear

13.	What is meant by consistency of greases?

14.	Write the generalized Reynolds equation for two-dimensional flow and indicate the different terms involved

15.	State the need for surface modification.

PART – C  (5 x 15 = 75 MARKS)

16.	Describe with appropriate sketches, the following methods of measuring friction:

a.	Inclined plane rig     	b.	Pin-on-disk rig

(OR)

17.	State the conditions under which the ploughing component of friction becomes dominant. Explain with sketches, how the ploughing component of friction is estimated using the following model rigid asperities: 	a.	conical 		b.	spherical

18.	List the different modes of plastic deformation by which material is removed in an abrasive wear situation.  Give schematic representation of each of them highlighting the salient aspects.

(OR)

19.	What is meant by ferrography? Explain the technique in detail.

20.	a.	What is meant by viscosity index of a lubricant? Indicate a method by which the viscosity

index of  a lubricating oil can be estimated   .

b.	Sketch the control circuit of a hydrostatic bearing and explain how it works

(OR)

[P.T.O]

21.	a.	Make a comparison between mineral lubricants and synthetic lubricants

b.	Explain the special features of gas lubrication

22.	A journal bearing has a diameter of 75 mm and is 75 mm long and supports a load of 8000 N. The journal speed is 600 rpm and the radial clearance is 0.0075 mm. Find the minimum oil-film thickness and the maximum film pressure for both SAE 10 and SAE 40 lubricants if the operating temperature is 600C. Discuss why one has a larger film thickness than the other.

(OR)

23.	Derive the expressions for the following for a Rayleigh step bearing starting from Reynolds equation

a.	pressure distribution (sketch the pressure profile) 	b.	load carrying capacity

24.	What is meant by micro structural treatment? List the different processes that belong to this category. Explain any three of them in detail.

(OR)

25.	State the advantages and the different types of vapor deposition processes. Explain any one of the processes in detail.