Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	MODERN VEHICLE TECHNOLOGY						Time : 3 hours

Subject Code:	ME276			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	In a four-stroke cycle engine, the operations namely suction, compression, expansion and 	exhaust are completed in the number of revolutions of crankshaft equal to -------------

2.	In----------- crankcase ventilation system fresh air supply is inducted into the crankcase during the compression stroke

3.	Another name for the steering link rod is-------------

4.	The important elements of a suspension system are -------------- and -----------

5.	------------------ is a hydraulic unit that replaces a clutch in a semi or fully automatic system, and transmits the engine torque to transmission system

6.	The hand brake usually operates on --------------

7.	The central gear of epicyclic gear set is called a -----------

8.	The smallest gears inside the differential casing are -----------------

9.	The ---------- are incorporated to measure the operating data at different locations

10	The Society of Automotive Engineers (SAE) has recommended that the selector should have the sequence ----------- in the case of a four-speed transmission system.

PART – B  (5 x 3 = 15 MARKS)

11.	Write short notes about the connecting rod in I.C engines

12.	List out the difference between thermo-syphon and forced cooling system.

13.	List out the requirements of a good braking system.

14.	Define propeller shaft with its neat sketch.

15.	List out the functions of the injectors.

PART – C  (5 x 15 = 75 MARKS)

16.	 Draw and explain about the different arrangement of engine cylinder.

(OR)

17.   Draw the sketch and explain the functions of splash lubricating system.

18.	 Draw and explain the working principle of power steering mechanism with its

neat sketch.

(OR)

19.	 Explain the differences between the radial and cross ply tyres with its sketches.

20.	 Draw the sketch and explain the working principle of Fluid coupling

(OR)

21.	 Draw the sketch and explain the working principle of Antilock Braking  System (ABS)

22.	 Explain the working principle of synchromesh gear with its neat sketch

(OR)

23.	Why differential is necessary? Explain the working principle of any one type

of differential with its neat sketch.

24.	 Explain the working principle of electronic engine control systems with its

various sensors.

(OR)

25.	Explain the working principle of common rail direct injection system with its

neat sketch.