Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – November / December 2008

Subject Title: 	ADVANCED THERMODYNAMICS					              Time : 3 hours

Subject Code: 	ME301			         						               Maximum Marks: 100

#### Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Air flows steadily at the rate of 0.5 kg/s through an air compressor, entering at 7 m/s velocity, 100 kPa pressure and 0.95 m3/kg volume, and leaving at 5 m/sec,700 kPa, 0.19 m3/kg. The internal energy of the air leaving is 90 kJ/kg greater than that of air entering. Cooling water in the compressor jackets absorbs heat from the air at the rate of 58 kW.

(a)	Compute the rate of shaft work input to the air in kW.

(b)	Find the ratio of the inlet pipe diameter to outer pipe diameter.

2.	Two kg of air at 500 kPa, 80° C expands adiabatically in a closed system until its volume is doubled and its temperature becomes equal to that of the surroundings which is at 100 kpa, 5° C. For this process, determine

(a) 	the maximum work

(b) 	the change in the availability

(c)    the irreversibility

For air, take Cv= 0.718 kJ/kg K, u = Cv T where cv is constant, and PV = mRT where p is pressure in kPa, V volume in m3, m mass in kg. R a constant equal to 0.287 kJ/kg K, and T temperature in K.

(OR)

3	Air is flowing steadily in an insulated duct. The pressure and temperature measurements of the air at two stations. A and B are given below. Establish the direction of the flow of air in the duct. Assume for air, specific heat Cp is constant at 1.005 kJ/kg-K, h = Cv T, and (V/T) =(0.287/P), where P,Vand T are pressure (in kPa), volume (in m3/kg) and temperature (in K) respectively.

Station A                 Station B

Pressure              130 kPa                    100 kPa

Temperature        50° C                         13° C

4.	Derive the equation (∂Cp/∂p) = -T (∂2V/∂ T2).

(OR)

5.	Explain the specific heat of a saturated vapour may be negative.

6.	A gasoline engine delivers 150 kW. The fuel used is C8H18  (l) and it enters the engine at 25° C.150 % theoretical air is used and it enters at 45° C. The products of combustion leave the engine at 750 K, and the heat transfer from the engine is 205 kW. Determine the fuel consumption per hour, if complete combustion is achieved.

(OR)

7.	Calculate the work of compression and heat transfer per kilogram when ethane is compressed reversibly and isothermally from 0.1 Mpa to 7 Mpa at a temperature of 45° C in a steady state, steady flow process.

8.	Estimate the heat transfer rate required to heat low pressure gaseous carbon tetrachloride (CCl4) from 500 to 1200 K in a steady state, steady flow, single inlet, single outlet, aergonic process at a flow rate of       1.0 kg/min.

(OR)

9.	Calculate the value of equilibrium constant Kp for the dissociation reaction

Na2            2Na at 1000° C. Given the energy of dissociation of Na2, Δεoo =0.73 eV, the fundamental vibration frequency  V of Na2is 159.23 cm-1 and the intermolecular distance is 3.078 Å.