Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ADVANCED HEAT AND MASS TRANSFER			        Time : 3 hours

Subject Code: 	ME320			         						                Maximum Marks: 100

Permitted to use of Heat and Mass Transfer Data book and Steam Table

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Consider steady two-dimensional heat transfer in a long solid bar of (a) square and (b) rectangular cross section as shown in figure. The measured temperatures at selected points of outer surfaces are shown. The thermal conductivity of the body is K = 20 W/mK, and there is no heat generation. Using the finite difference method with a mesh size of x = y = 1 cm, determine the temperatures at the indicated points in the medium.

150

180

200

100

120

120

100C

Insulation

1

2

1

2

180

120

120

3

4

32

4

200

140

140

Insulation

a. 	Square						b.	Rectangular

2.	a.	Consider a long pipe of inner radius r1, outer radius r2, and thermal conductivity k. The outer

surface of the pipe is subjected to convection to a medium at T with a heat transfer coefficient of h, but the direction of heat transfer is not known. Express the convection boundary condition on the outer surface of the pipe.								(14)

b.	Consider a hot backed potato on a plate. The temperature of the potato is observed to drop

by 4C during the first minute. Will the temperature drop during the second minute be less than, equal to, or more than 4C? Why?											(6)

(OR)

3.	Derive the general 3-dimensional heat conduction equation in cylindrical coordinators.

4.	a.	The convective heat transfer coefficient for a clothed person standing in moving air is 	expressed as h = 15V0.69 for 0.15 < V < 1.5 m/s, where V is the air velocity. For a person with a 	body surface area of 1.7 m2 and an average surface temperature of 20C, determine the rate of 	heat loss from the person in windy air at 10C by convection for air velocities of (i) 0.5 m/s      	(ii) 1 m/s  (iii) 1.5 m/s.												(12)

[P.T.O]

b.	Engine oil at 60C flows over the upper surface of a 5m long flat plate whose temperature is

20C with a velocity of 2 m/s. Determine the total drag force and the rate of heat transfer per unit width of the entire plate.													(8)

(OR)

5.	A hot plate 30 cm high and 1.2 cm wide at 140C is exposed at ambient air at 20C. Using the approximate solution, calculate the following (i) Maximum velocity at 12 cm from the leading edge of the plate (ii) boundary Layer thickness at 12 cm from the leading edge of plate (iii) local heat transfer coefficient at 12 cm from the leading edge of the plate (iv) average heat transfer coefficient over the surface of the plate (v) total heat loss from plate (vi) temperature rise of air.

6.	A furnace is shaped like a long equilateral triangular duct, as shown in Fig. The width of each side is 1 m. The base surface has an emissivity of 0.7 and is maintained at a uniform temperature of 600K. The heated left-side surface closely approximates a blackbody at 1000K. The right-side surface is well insulated. Determine the rate of which heat must be supplied to the heated side externally per unit length of the duct in order to maintain these operating conditions.

3

2

T2 = 1000k

Black

2

1 = 0.7

T1 = 600 k

(OR)

7.	A spherical tank with diameter D1 = 40 cm, filled with cryogenic fluid at T1 = 100 K, is placed inside a spherical container of diameter D2 = 60 cm. maintained at T2 = 300K. Emissivities of inner and outer tanks are 1 = 0.1 and 2 = 0.2 respectively. Find (i) the rate of heat loss into the inner vessel by radiation (ii) If the spherical radiation shield of diameter  D3 = 50 cm, with an emissivity 3 = 0.05 on both surfaces is placed between the spheres, what is the new rate of heat loss?

8.	A gas mixture consists of oxygen and nitrogen at 1 bar and 27C. The oxygen content by volume at two planes 3 mm apart are 15% and 30% respectively. Calculate the rate of diffusion in kg mole / (sm2), if (i) Nitrogen is non-diffusing (ii) there is equimolal counter-diffusion of the two gases. Take D = 0.181 cm2/s.

(OR)

9.	a.	Write a short notes on diffusion coefficient in a binary mixture of (i) gases (ii) liquids and

(iii) solids.																	(12)

b.	Explain the (i) Schmidt and Lewis numbers.										(8)