Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	FINITE ELEMENTS ANALYSIS					              Time : 3 hours

Subject Code: 	ME346			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

A long bar of rectangular cross section, having thermal conductivity of 1.5 Wm-1 C-1 is subjected to the boundary conditions shown in figure. Two opposite sides are maintained at a uniform temperature of 180C; one side is insulated, and the remaining side is subjected to a convection process with T=25C and h=50Wm-2 C-1. Determine the temperature distribution in the bar.

T=1800C

h=50 W/m2 0C

0.6 m

K= 1.5 W/m 0C

q=0

T=25C

0.4 m

T=1800C

2.	Consider the bar shown in figure . An axial load P=200 x 103N is applied as shown in figure. Using the penalty approach for handling boundary conditions, do the following;

a.	Determine the nodal displacement.

b.	Determine the stress in each material.

c.	Determine the reaction forces.

300 mm

400 mm

``

P

X

1

2

3

1

2

Aluminum

A1= 2400 mm2

E1= 70 x 109 N/m2

Steel

A2= 600 mm2

E2= 200 x 109 N/m2

(OR)

3.	Solve the simultaneous linear algebraic equations using Gaussian Elimination method.

X1 – X2 + 3X3 = 10

2X1 + 3X2 + X3 = 15

4X1 + 2X2 – X3 = 6.

4.	Evaluate the shape functions N1, N2 and N3 at the interior point P for the triangular element shown in figure. Determine the strain displacement matrix.

y

3(4, 7)

2(7 , 3.5)



P(3.85, 4.8)

x

1(1.5 , 2)

(OR)

5.	Derive the shape functions and force vectors for 1D and 2D simplex element using global coordinate system.

φ(x)

0

φ(x)

φi

i

φ(x)=α1+α2x

x

xi

l=(xj−xi)

xj

φj

x

j

1D

2D

x

y

φ(x,y)

φ(x,y)=α1+α2x+α3y

φi

(x,y)

φj

φk

i(xi,yi)

k(xk,yk)

k(xk,yk)

6.	For the beam and loading shown in figure. Determine (a) the slope at 2 and 3 (b) the vertical deflection at the mid point of the distributed load.

12 kN/m

2

1

2

3

1

1m

1m

E= 200 GPa

I=4 x 106 mm4

(OR)

7.	Explain 3 Dimensional stress strain relations for isotropic materials.

8.	Compute the element matrices and vectors for the element shown in figure. when the edges jk and ki experience convection heat loss.

j

y

(8,10)

h=10W/cm2K

T=400C

k=60W/cm K

q0=50 W/cm3

i

k

(4,6)

(12,8)

x

h=15W/cm2K

T=400C

(OR)

9.	Find the temperature distribution in the one dimensional fin shown in fig.

h=5Wattscm2k

T=40C

1 cm radius

140C

End surface A

L = 5 cm

k=70Wattscm2k