Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ADVANCED STRENGTH OF MATERIALS			              Time : 3 hours

Subject Code: 	ME353			         						                Maximum Marks: 100

Answer ALL questions (5 x 20 = 100 Marks)

1.	Compulsory:

Determine the total strain energy of the steel beam shown in fig :1. The cross section is

rectangular, 25mm wide and 50mm deep.

Take : Es = 205 GPa and  = 0.3

12 kN

y

50mm

x

0.5m

1m

2.	Determine the principal stresses and directional cosines associated with normal to the surfaces of

each principal stress, for the stress matrix given below.

σ=311102120

(OR)

3.	Determine the displacement field due to the bending. Consider the cross section of the beam to be rectangular and thin so that deflections are not functions of Z. Stiffness of the beam is EIz and Poisson’s ratio .

P

L

X

4.	Estimate the torque on a 10mm diameter steel shaft when yielding begin (a) The Tresca and (b) the Von Mises theory. The yield strength of the steel is 140 MPa.

(OR)

5.	a.	Determine the maximum shear stress on the outer surface of a closed thin walled circular

cylindrically cylinder internally pressurized at 7 MPa. The mean dia of the cylinder is 500mm and wall thickenss is 12mm.

b.	Explain Tresca and Von Mises theory for Ductile materials.

[P.T.O]

6.	Estimate shear stress and total angle of twist of thin walled circular of outer diameter 125mm and

thickness 5mm. The steel tube is 0.5m long and transmitting a torque of 1 kN-m. The material constant are E = 200 GPa and  = 0.29

(OR)

7.	A beam section composed of aluminium (Ea = 0.7 x 105 N/ mm2 and steel (Es = 2 x 105 N/mm2) are bonded together as shown in figure. If the section is undergoing a positive bending moment of Mz = 1 x 106 N.mm about a horizontal axis. Determine the resulting stress distribution.

50mm

50mm

25mm

8.	Determine the stress fields that arise from the following stress functions

a.	 = cy2			b.	 = Ax2 + Bxy + cy2

c.	 = Ax3 + Bx2y + Cxy2 + Dy3

where A, B, C and D are constant.

(OR)

9.	Consider the equilateral cross section with sides of length ‘a’ shown in figure. Determine the shear stress distribution if the section is transmitting a torque T.

Z

h3

h=32a

a