Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	ELECTRONIC DEVICES AND BASIC CIRCUITS			Time : 3 hours

Subject Code:	MT224			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	What is a signal?

2.	Define bandwidth of an amplifier.

3.	The minimum number of terminals needed by a single op-amp is…………. and by integrated circuits with four op-amps is ………

4. 	Define CMRR.

5.	Define peak inverse voltage.

6. 	Draw the V-I characteristics of ideal diode.

7.	What do you mean by avalanche breakdown?

8.	What is the difference between NMOS and PMOS?

9. 	Draw the circuit symbol of NPN transistor.

10.	State Miller’s theorem.

PART – B  (5 x 3 = 15 MARKS)

11.	Write about the four gain parameters of an amplifier?

12.	Explain functions and characteristics of the ideal op-amp..

13.	Write about photodiode.

14.	Write about operation of MOSFET as linear amplifier.

15.	Explain the high frequency response of CE amplifier.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain in detail about the cascaded amplifier.

(OR)

17.	Write about Frequency spectrum of signals.

18.	Explain the op-amp as Integrator and Differentiator.

(OR)

19.	Explain DC imperfections of Op – amp.

20.	Explain full wave rectifier in detail

(OR)

21.	Describe in detail about the clamper circuits.

22.	Explain the depletion type MOSFET in detail.

(OR)

23.	Consider a process technology for which Lmin=0.4µm,tox=8nm, µn=450cm2/V.s and Vt=0.7V.

a.	find Cox and k’n

b.	for a MOSFET with W/L= 8 µm/.8 µm, calculate the values of Vgs Vdsmin needed to operate the transistor in the saturation region with the dc current Id=100 µA.

[P.T.O]

c.	For the device in (b) , find the value of Vgs required to cause the device to operate as a 1000Ω resistor for very small Vds.

24.	Explain DC biasing of BJT amplifier.

(OR)

25.	A transistor amplifier is fed with a signal source having open circuit voltage vsig of 10mV and internal resistance Rsig of 100kΩ . The voltage vi at the amplifier input and the output voltage vo measured both without and with the load resistor RL = 10KΩ connected to the output of amplifier. The measured results are:

Without RL:  vi=9mV,vo=90mV and with RL:  vi=8mV,vo=70mV.

Find Avo, Av, Gvo, Ri  and Ro the parameters of the amplifier