Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	APPLICATION OF DSP FOR AUDIO AND IMAGE PROCESSING													               						Time : 3 hours

Subject Code: 	MT246			         						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	Mention some commercial applications of DSP.

2.	List the parameters that determine the operating range of a radar system?

3.	What are the three parts of a continues sound?

4. 	How does fricative sounds originate?

5.	What are the two adjustable parameters of a lens system?

6.	Mention some uses of warping?

7.	When the spatial domain is continuous, the frequency domain is ______________.

8.	Name some iterative techniques used in CT reconstruction?

9.	Name some lossless compression techniques.

10.	What is an I picture frame.

PART – B  (5 x 3 = 15 MARKS)

11. 	How is artificial reverberation done?

12. 	Define timbre?

13. 	Define Sampling aperture in an image.

14. 	What is a quantum sink?

15. 	Mention some advantages of Fourier transform.

PART – C  (5 x 15 = 75 MARKS)

16.	Explain how DSP helps in multiplexing, compression and echo control in telecommunications.

(OR)

17.	Explain the applications of DSP in the fields of

(a.) Radar  (b.) Computed Tomography.

18.	Define ‘Audio Data Rate’. Explain how digitized audio signal depends on the data rate.

(OR)

19.	Explain the non linear audio processing techniques.

20.	Using suitable diagrams compare and explain image formation in a human eye and an electronic camera.

(OR)

21.	Explain image convolution methods in detail.

22.	Explain how illumination flattening is done using a large PSF.

(OR)

23. 	Explain the various techniques used to measure spatial resolution of images.

24	Discuss about the various lossless and lossy compression techniques in detail.

(OR)

25. 	Encode 123  145  201  4  119  89  243  245  59  11  206  145  201  4  243  245 using LZW algorithm. Draw the code table also.