Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	METROLOGY AND COMPUTER AIDED INSPECTION		Time : 3 hours

Subject Code:	PE227			       						                       Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	An accurate instrument will always be precise  (True or False)

2.	Name a legal agency that can prescribe standards

3.	Define least count of a vernier

4.	Write down the Indian Standards Institution specification for vernier calipers

5.	The longer wavelength irregularities upon which roughness is superimposed is termed as ---------

6.	Mention TWO methods for measuring straightness

7.	What is a transmission thread?

8.	Mention three methods of specifying pitch of gears

9.	What is a Co ordinate Measuring Machine?

10.	Differentiate between on line and off line inspection

PART – B  (5 x 3 = 15 MARKS)

11.	Differentiate between ‘Precision’ and ‘Accuracy’

12.	Differentiate between unilateral and bilateral tolerances

13.	With a sketch indicate ‘run out’ of a rotating round component

14.	Mention any six important elements of parallel screw threads

15.	Mention the three basic functions of a machine vision system

PART – C  (5 x 15 = 75 MARKS)

16.	a.	What is measurement? What are the requirements for a measurement procedure? (5 marks)

b.	Define the following terms 	i.	Accuracy 	ii.	Precision	 iii.	Uncertainty 				iv.	Sensitivity 	v.	Readability   										(5x2 = 10)

(OR)

17.	a.	Define ‘error’ in measurement. Outline how errors are classified? (7 marks)

b.	What is ‘calibration’? What are the requirements during a calibration procedure? Outline

the steps involved in calibration 												(8)

18.	a.	What are gauge blocks? How gauge blocks are classified? Outline the procedure for

making a stake of gauge blocks( 8 marks)

b.	Outline the precautions to be observed to preserve the accuracy of gauge blocks 		(7)

(OR)

19.	a.	What are comparators? What are the basic difference between comparators and measuring

Instruments? 																(3)

b.	With a neat sketch outline the working of the Johansson ‘Mikrokator’ 				(12)

20.	a.	Define straightness of a line in two planes. With a sketch illustrate the tolerance on the

straightness of a line 															(5)

b.	With a neat sketch describe the spirit level method of straightness testing 			(10)

(OR)

[P.T.O]

21.	a.	What is surface texture? With neat sketches illustrate the elements of surface texture of a

machined surface 															(5)

b.	With a neat sketch outline the working of the Tomlinson surface meter				 (10)

22.	a.	Define the following with reference to a parallel thread 							(3)

i. 		Pitch 		ii. 		lead 		iii.		helix angle

b.	Describe the procedure for measuring the effective diameter of a parallel screw using the

three wire method 															(12)

(OR)

23.	a.	Mention 6   important elements to be checked during spur gear measurement 		(3)

b.	With a neat sketch describe the method of measuring gear tooth thickness using gear

tooth vernier 																(12)

24.	a.	With a neat sketch describe the construction of a coordinate measuring machine         (8)

b.	Outline the applications of Coordinate Measuring Machine                                          (7)

(OR)

25.	Write notes on the following  												(3x5 =15)

a.	Vision systems in metrology	b.	laser interferometers 	c.	In process inspection