Reg. No. ________

Karunya University

(Declared as Deemed to be University under Sec.3 of the UGC Act, 1956)

End Semester Examination – May / June 2009

Subject Title: 	PRODUCTION PROCESS									Time : 3 hours

Subject Code:	PE240				       						                Maximum Marks: 100

#### Answer ALL questions

PART – A  (10 x 1 = 10 MARKS)

1.	The term machinability refers ---------------------------

2.	How can you specify size of a lathe?

3.	The role of flute in drill tool is -----------------------

4.	Give any two examples for the natural abrasives.

5.	Write any two mechanical energy based non conventional machining process

6.	Define DNC system

7.	Extrusion ratio is defined as ------------------

8.	Classify the HERF process

9.	Cold shut occur in casting due to --------------------

10.	What do you mean by straight polarity in arc welding process

PART – B  (5 x 3 = 15 MARKS)

11.	Find the angle at which the compound rest should be set up to turn taper on work  piece having a length of 200 mm larger diameter 45 mm and smaller diameter 30 mm.

12.	How can you select a grinding wheel?

13.	Explain the closed loop system in an NC machine

14.	Write about the drawing process

15.	Discuss the properties of moulding sand

PART – C  (5 x 15 = 75 MARKS)

16.	With a neat sketch explain in detail how the thread cutting operation can be Performed in Lathe

(OR)

17.	Draw the profile of the milling cutter and give its nomenclature in detail

18.	Explain the any one type quick return mechanism in shaper with neat sketch

(OR)

19.	What do you under stand dressing  of grinding wheel?  Explain the cylindrical grinding machine

20.	Write in detail on the following

a.	Ultra sonic machining		b.	Laser Beam welding				(7.5+7.5)

(OR)

21.	Explain the working principle of NC machine tool.

22.	Derive an expression for bending force and give the nomenclature of bending

(OR)

23.	Explain the principle and working of explosive forming process

24.	Explain the shell casting and investment casting with neat sketch

(OR)

25.	Write short notes on

a.	Submerged arc welding 	b.	Gas welding 									(7.5+7.5)